For ode45

#include <dace/dace.h>

#include <cmath>

using namespace std;

using namespace DACE;

//// Two-body problem (TBP) function

template<typename T> AlgebraicVector<T> TBP(AlgebraicVector<T> x, double t)

{

AlgebraicVector<T> pos(3), res(6);

pos[0] = x[0];

pos[1] = x[1];

pos[2] = x[2];

T r = pos.vnorm();//used for finding magnitude

const double mu = 398600; // km^3/s^2

res[0] = x[3]; // velocity in x direction

res[1] = x[4]; // velocity in y direction

res[2] = x[5]; // velocity in z direction

// acceleration due to gravity

res[3] = -mu * pos[0] / (r * r * r);

res[4] = -mu * pos[1] / (r * r * r);

res[5] = -mu * pos[2] / (r * r * r);

return res;

}

// Runge-Kutta 4th Order method

template<typename T> AlgebraicVector<T> RK4(AlgebraicVector<T> x, double t0, double t1)

{

const double hmax = 0.1;

int steps = ceil((t1 - t0) / hmax);

double h = (t1 - t0) / steps;

double t = t0;

for (int i = 0; i < steps;

i++)

{

// Compute the 4 Runge-Kutta stages

AlgebraicVector<T> k1 = TBP(x, t);

AlgebraicVector<T> k2 = TBP(x + 0.5 * h * k1, t + 0.5 * h);

AlgebraicVector<T> k3 = TBP(x + 0.5 * h * k2, t + 0.5 * h);

AlgebraicVector<T> k4 = TBP(x + h * k3, t + h);

// Update x for the next step using RK4 formula

x = x + (h / 6.0) * (k1 + 2.0 * k2 + 2.0 * k3 + k4);

// Increment time

t += h;

}

return x;

}

int main(void)

{

// Initialize DACE (Differential Algebra Core Engine)

DA::init(3, 6);

// Declare initial and final state vectors

AlgebraicVector<DA> x0(6), xf(6);

// Set initial conditions

const double mu = 398600;

const double ecc = 0.1;

// Initial position and velocity in DA form

x0[0] = -1871.0 + DA(1); // 1000 km altitude

x0[1] = 10639 + DA(2);  // position along y axis

x0[2] = 532 + DA(3);     // position along z axis

x0[3] = -6.2506 + DA(4); // initial velocity in x

x0[4] = -1.0035 + DA(5); // velocity in y

x0[5] = .1182 + DA(6); // velocity in z

// Integrate for half the orbital period

const double pi = 4.0 * atan(1.0);

double a = 12000; // semi-major axis

// Final state using RK4

//xf = RK78( 6, x0, 0.0, pi*sqrt(a*a*a/mu), rhs );

xf = RK4(x0, 0.0, 2*pi * sqrt(a * a * a / mu));

//Print initial and final conditions

cout << endl << "Initial conditions:" << endl << endl;

cout << x0 << endl << endl;

cout << endl << "Final conditions:" << endl << endl;

cout << xf.cons() << endl;

}