int main() {

// **1. Initialize DACE**

struct Velocity {

double vx, vy, vz;

};

DA::setEps(0.0);

const int filterorder = 2;

const int nvar = 6;

DA::init(2 * filterorder, nvar);

// **2. Orbital Parameters**

const double mu = 398600.0;

const double pi = 3.14159265358979323846;

const double a = 8788.0;

const double T = 2.0 * pi * sqrt(pow(a, 3) / mu);

const int nObsPerIter = 24;

const double dtspan = T / nObsPerIter;

const int nmeas = 3;

// Measurement noise covariance [R]

const double sigmaRpos = 1e-2;

const double sigmaRang = (10 / 3600.0) * pi / 180.0;

AlgebraicMatrix<double> R(nmeas, nmeas);

for (int i = 0; i < nmeas; i++)

{

for (int j = 0; j < nmeas; j++) { R.at(i, j) = 0.0; }

}

R.at(0, 0) = sqr(sigmaRpos);

R.at(1, 1) = sqr(sigmaRang);

R.at(2, 2) = sqr(sigmaRang);

// Process noise covariance [Q]

AlgebraicMatrix<double> Q(nvar, nvar);

const double sigmaQ = 0.0;

for (int i = 0; i < nvar; i++)

{

for (int j = 0; j < nvar; j++) { Q.at(i, j) = 0.0; }

}

for (int i = 0; i < nvar; i++) { Q.at(i, i) = sqr(sigmaQ); }

// Scaled measurement noise

double sigmaPpos = 1.0;

double sigmaPang = sigmaPpos / a;

AlgebraicMatrix<double> RScaled(nmeas, nmeas);

for (int i = 0; i < nmeas; i++) for (int j = 0; j < nmeas; j++) RScaled.at(i, j) = 0.0;

RScaled.at(0, 0) = R.at(0, 0);

RScaled.at(1, 1) = R.at(1, 1);

RScaled.at(2, 2) = R.at(2, 2);

// **3. Noise Distributions**

std::default_random_engine generator;

generator.seed(100);

std::normal_distribution<double> position_distribution(0.0, 87.88);

std::normal_distribution<double> velocity_distribution(0.0, 6.74e-4);

std::normal_distribution<double> distribution_r(0.0, sigmaRpos);

std::normal_distribution<double> distribution_ang(0.0, sigmaRang);

// **4. Initial Conditions**

AlgebraicVector<DA> x0(nvar);

AlgebraicVector<double> x0_double(nvar);

x0[0] = -6045.0; x0[1] = -3490.0; x0[2] = 2500.0;

x0[3] = -3.457;  x0[4] = 6.618;   x0[5] = 2.533;

x0_double = cons(x0);

// **5. Model Setup - Modified with azimuth and elevation combinations**

// Compute velocity perturbations

const double V = 0.3; // km/s

std::vector<Velocity> delta_v_combinations;

std::vector<double> azimuth = { 0, 90, 180, 270 };

std::vector<double> elevation = { -90, -45, 0, 45, 90 };

for (double theta_deg : azimuth) {

for (double phi_deg : elevation) {

double theta = theta_deg * pi / 180.0;

double phi = phi_deg * pi / 180.0;

Velocity v;

v.vx = V * cos(theta) * cos(phi);

v.vy = V * sin(theta) * cos(phi);

v.vz = V * sin(phi);

delta_v_combinations.push_back(v);

}

}

const int n_models = delta_v_combinations.size();

std::vector<Velocity> model_impulses = delta_v_combinations;

Velocity true_impulse = { 0.0, 0.0, +0.3 };

// **6. Setup for Monte Carlo Simulation**

const int MC = 50;

double err_sum[nvar][nObsPerIter] = { 0 };

double err_sq_sum[nvar][nObsPerIter] = { 0 };

double cov_sum[nvar][nObsPerIter] = { 0 };

// Open Output Files

std::ofstream error_files[6];

std::string error_names[6] = { "error_x.csv", "error_y.csv", "error_z.csv",

"error_Vx.csv", "error_Vy.csv", "error_Vz.csv" };

std::ofstream mean_files[6];

std::string mean_names[6] = { "mean_x.csv", "mean_y.csv", "mean_z.csv",

"mean_Vx.csv", "mean_Vy.csv", "mean_Vz.csv" };

std::ofstream num_3sigma_files[6];

std::string num_3sigma_names[6] = { "num_3sigma_x.csv", "num_3sigma_y.csv", "num_3sigma_z.csv",

"num_3sigma_Vx.csv", "num_3sigma_Vy.csv", "num_3sigma_Vz.csv" };

std::ofstream cov_3sigma_files[6];

std::string cov_3sigma_names[6] = { "cov_3sigma_x.csv", "cov_3sigma_y.csv", "cov_3sigma_z.csv",

"cov_3sigma_Vx.csv", "cov_3sigma_Vy.csv", "cov_3sigma_Vz.csv" };

std::ofstream weights_file("model_weights.csv");

// **New CSV Files for P_final 3-Sigma Values**

std::ofstream P_final_3sigma_files[6];

std::string P_final_3sigma_names[6] = { "P_final_3sigma_x.csv", "P_final_3sigma_y.csv", "P_final_3sigma_z.csv",

"P_final_3sigma_Vx.csv", "P_final_3sigma_Vy.csv", "P_final_3sigma_Vz.csv" };

// Initialize all output files

for (int i = 0; i < 6; i++) {

error_files[i].open(error_names[i]);

mean_files[i].open(mean_names[i]);

num_3sigma_files[i].open(num_3sigma_names[i]);

cov_3sigma_files[i].open(cov_3sigma_names[i]);

P_final_3sigma_files[i].open(P_final_3sigma_names[i]);  // Open new files

if (!error_files[i].is_open() || !mean_files[i].is_open() ||

!num_3sigma_files[i].is_open() || !cov_3sigma_files[i].is_open() ||

!P_final_3sigma_files[i].is_open()) {

std::cerr << "Error opening file for state " << i << std::endl;

return 1;

}

error_files[i] << std::scientific; error_files[i].precision(15);

mean_files[i] << std::scientific; mean_files[i].precision(15);

num_3sigma_files[i] << std::scientific; num_3sigma_files[i].precision(15);

cov_3sigma_files[i] << std::scientific; cov_3sigma_files[i].precision(15);

P_final_3sigma_files[i] << std::scientific; P_final_3sigma_files[i].precision(15);

}

weights_file << "MC_run,time";

for (int m = 0; m < n_models; m++) weights_file << "," << "model" << m << "_weight";

weights_file << "\n";

generator.seed(100);

// Per-model variables

std::vector<AlgebraicVector<double>> mn_model(n_models, x0_double);

std::vector<AlgebraicMatrix<double>> PScaled_model(n_models, AlgebraicMatrix<double>(nvar, nvar));

std::vector<AlgebraicVector<DA>> xc_current_model(n_models, AlgebraicVector<DA>(nvar));

// **7. Main Loop - Monte Carlo Simulation**

for (int k = 0; k < MC; k++) {

std::cout << "Run Number: " << k + 1 << "/" << MC << std::endl;

// initialize initial error covence matrdix

// In Monte Carlo loop, replace initial P initialization

AlgebraicMatrix<double> P(nvar, nvar);

for (int i = 0; i < nvar; i++) {

for (int j = 0; j < nvar; j++) {

P.at(i, j) = 0.0;

}

if (i < 3) {

P.at(i, i) = pow(87.88, 2); // Position variance: (87.88 km)^2

}

else {

P.at(i, i) = pow(6.74e-4, 2); // Velocity variance: (6.74e-4 km/s)^2

}

}

//AlgebraicMatrix<double> P(nvar, nvar);

//for (int i = 0; i < nvar; i++) {

//    for (int j = 0; j < nvar; j++) {

//        P.at(i, j) = (i == j) ? pow(.02 * abs(x0_double[i]), 2) : 0.0;//      2 percentge of stste value

//    }

//}

// Initialize per-model covariances and states

// AlgebraicMatrix<double> P(nvar, nvar);

for (int m = 0; m < n_models; m++) {

PScaled_model[m] = P + Q;

mn_model[m] = x0_double;

}

// Apply noise to initial state

AlgebraicVector<double> delta_x(nvar);

for (int c = 0; c < 3; c++) delta_x[c] = position_distribution(generator);

for (int c = 3; c < 6; c++) delta_x[c] = velocity_distribution(generator);

AlgebraicVector<double> current_mc = x0_double + delta_x;

std::vector<double> weights(n_models, 1.0 / n_models);

double t = 0.0;

// Observation loop

for (int obs = 0; obs < nObsPerIter; obs++) {

double tk = t + dtspan;

DA::pushTO(filterorder);

current_mc = RK78(nvar, current_mc, t, tk, TBP<double>);

if (obs == nObsPerIter / 2) {

current_mc[3] += true_impulse.vx;

current_mc[4] += true_impulse.vy;

current_mc[5] += true_impulse.vz;

}

t = tk;

DA::popTO();

const auto& st = current_mc;

AlgebraicVector<double> ztrue(nmeas);

ztrue[0] = sqrt(st[0] * st[0] + st[1] * st[1] + st[2] * st[2]) + distribution_r(generator);

ztrue[1] = atan2(st[1], st[0]) + distribution_ang(generator);

double arg = st[2] / ztrue[0];

arg = std::max(-1.0, std::min(1.0, arg));

ztrue[2] = asin(arg) + distribution_ang(generator);

std::vector<double> likelihoods(n_models, 0.0);

double weight_sum = 0.0;

const double epsilon = 1e-300;

for (int m = 0; m < n_models; m++) {

// Initialize DA state

for (int i = 0; i < nvar; i++) {

xc_current_model[m][i] = mn_model[m][i];

xc_current_model[m][i] += DA(i + 1);

}

DA::pushTO(filterorder);

xc_current_model[m] = RK78(nvar, xc_current_model[m], t - dtspan, t, TBP<DA>);

DA::popTO();

// Apply model-specific impulse at midpoint

AlgebraicVector<double> mn_prop(nvar, 0.0);

if (obs == nObsPerIter / 2) {

mn_prop[3] = model_impulses[m].vx;

mn_prop[4] = model_impulses[m].vy;

mn_prop[5] = model_impulses[m].vz;

}

DA::pushTO(2 * filterorder);

DA PscaledPol = 0;

for (int i = 0; i < nvar; i++) {

for (int j = i; j < nvar; j++) {

PscaledPol += PScaled_model[m].at(i, j) * DA(i + 1) * DA(j + 1);

}

}

DA moments = computeMoments(PscaledPol);

AlgebraicVector<DA> xf = xc_current_model[m];

AlgebraicVector<DA> zf(nmeas);

zf[0] = sqrt(xf[0] * xf[0] + xf[1] * xf[1] + xf[2] * xf[2]);

zf[1] = atan2(xf[1], xf[0]);

zf[2] = asin(xf[2] / zf[0]);

AlgebraicVector<double> zpred(nmeas);

for (int i = 0; i < nmeas; i++) {

zpred[i] = zf[i].evalMonomials(moments);

}

for (int i = 0; i < nvar; i++) {

mn_model[m][i] = xf[i].evalMonomials(moments);

}

mn_model[m] = mn_model[m] + mn_prop;

for (int i = 0; i < nvar; i++) xf[i] = xf[i] - mn_model[m][i];

for (int i = 0; i < nmeas; i++) zf[i] = zf[i] - zpred[i];

AlgebraicMatrix<DA> xx(nvar, nvar), zz(nmeas, nmeas), xz(nvar, nmeas);

AlgebraicMatrix<double> PUPD(nvar, nvar), PZZ(nmeas, nmeas), PXZ(nvar, nmeas);

for (int i = 0; i < nvar; i++) {

for (int j = i; j < nvar; j++) {

xx.at(i, j) = xf[i] * xf[j];

PUPD.at(i, j) = xx.at(i, j).evalMonomials(moments);

PUPD.at(j, i) = PUPD.at(i, j);

}

}

for (int i = 0; i < nmeas; i++) {

for (int j = i; j < nmeas; j++) {

zz.at(i, j) = zf[i] * zf[j];

PZZ.at(i, j) = zz.at(i, j).evalMonomials(moments);

PZZ.at(j, i) = PZZ.at(i, j);

}

}

PZZ = PZZ + RScaled;

for (int i = 0; i < nmeas; i++) PZZ.at(i, i) += 0; // Regularize

for (int i = 0; i < nvar; i++) {

for (int j = 0; j < nmeas; j++) {

xz.at(i, j) = xf[i] * zf[j];

PXZ.at(i, j) = xz.at(i, j).evalMonomials(moments);

}

}

AlgebraicMatrix<double> INVPZZ = PZZ.inv();

AlgebraicMatrix<double> Kgain = PXZ * INVPZZ;

AlgebraicVector<double> zdiff = ztrue - zpred;

AlgebraicVector<double> mn_update(nvar, 0.0);

for (int i = 0; i < nvar; i++) {

for (int j = 0; j < nmeas; j++) {

mn_update[i] += Kgain.at(i, j) * zdiff[j];

}

}

mn_model[m] = mn_model[m] + mn_update;

PScaled_model[m] = PUPD - Kgain * PZZ * Kgain.transpose();

for (int i = 0; i < nvar; i++) {

for (int j = 0; j < i; j++) {

PScaled_model[m].at(i, j) = PScaled_model[m].at(j, i);

}

if (PScaled_model[m].at(i, i) < 0) PScaled_model[m].at(i, i) = 0; // Clamp

}

double exponent = 0.0;

for (int i = 0; i < nmeas; i++) {

for (int j = 0; j < nmeas; j++) {

exponent += zdiff[i] * INVPZZ.at(i, j) * zdiff[j];

}

}

exponent *= -0.5;

likelihoods[m] = std::max(exp(exponent), epsilon);

weights[m] *= likelihoods[m];

weight_sum += weights[m];

DA::popTO();

}

if (weight_sum <= 0.0) {

for (int m = 0; m < n_models; m++) weights[m] = 1.0 / n_models;

}

else {

for (int m = 0; m < n_models; m++) weights[m] /= weight_sum;

}

weights_file << k << "," << t;

for (int m = 0; m < n_models; m++) weights_file << "," << weights[m];

weights_file << "\n";

// Compute final weighted estimate

AlgebraicVector<double> mn_final(nvar, 0.0);

for (int m = 0; m < n_models; m++) {

for (int i = 0; i < nvar; i++) {

mn_final[i] += weights[m] * mn_model[m][i];

}

}

AlgebraicMatrix<double> P_final(nvar, nvar, 0.0);

for (int m = 0; m < n_models; m++) {

AlgebraicVector<double> diff = mn_model[m] - mn_final;

AlgebraicMatrix<double> diff_outer(nvar, nvar);

for (int i = 0; i < nvar; i++) {

for (int j = 0; j < nvar; j++) {

diff_outer.at(i, j) = diff[i] * diff[j];

}

}

P_final = P_final + weights[m] * (PScaled_model[m] + diff_outer);

}

// **Write 3-Sigma Values of P_final to CSV Files**

for (int i = 0; i < nvar; i++) {

double sigma = sqrt(std::max(0.0, P_final.at(i, i)));

P_final_3sigma_files[i] << 3 * sigma;

if (obs < nObsPerIter - 1) P_final_3sigma_files[i] << ",";

else P_final_3sigma_files[i] << "\n";

}

// Existing code for writing errors and other outputs

for (int i = 0; i < nvar; i++) {

double error = current_mc[i] - mn_final[i];

err_sum[i][obs] += error;

err_sq_sum[i][obs] += error * error;

error_files[i] << error;

double variance = std::max(0.0, P_final.at(i, i));

cov_sum[i][obs] += sqrt(variance);

if (obs < nObsPerIter - 1) error_files[i] << ",";

else error_files[i] << "\n";

}

}

}

// **8. Calculate and Write Statistics**

for (int i = 0; i < nvar; i++) {

for (int obs = 0; obs < nObsPerIter; obs++) {

double mean_err = err_sum[i][obs] / MC;

double variance = err_sq_sum[i][obs] / MC - mean_err * mean_err;

if (variance < 0) variance = 0;

double std_dev = sqrt(variance);

double num_3sigma = 3 * std_dev;

double cov_3sigma = 3 * (cov_sum[i][obs] / MC);

mean_files[i] << mean_err;

num_3sigma_files[i] << num_3sigma;

cov_3sigma_files[i] << cov_3sigma;

if (obs < nObsPerIter - 1) {

mean_files[i] << ",";

num_3sigma_files[i] << ",";

cov_3sigma_files[i] << ",";

}

else {

mean_files[i] << "\n";

num_3sigma_files[i] << "\n";

cov_3sigma_files[i] << "\n";

}

}

}

// **9. Clean Up**

for (int i = 0; i < nvar; i++) {

error_files[i].close();

mean_files[i].close();

num_3sigma_files[i].close();

cov_3sigma_files[i].close();

P_final_3sigma_files[i].close();  // Close new files

}

weights_file.close();

std::cout << "Simulation complete. Data and statistics written to CSV files.\n";

return 0;

}