int main() {

struct Velocity {

double vx, vy, vz;

};

// **1. Initialize DACE**

DA::setEps(0.0);

const int filterorder = 2;

const int nvar = 6;

DA::init(2 * filterorder, nvar);

// **2. Orbital Parameters**

const double mu = 398600.0; // Gravitational parameter (km^3/s^2)

const double pi = 3.14159265358979323846;

const double a = 8788.0; // Semi-major axis (km)

const double T = 2.0 * pi * sqrt(pow(a, 3) / mu); // Orbital period (s)

const int nObsPerIter = 48; // Number of observation steps

const double dtspan = T / nObsPerIter; // Time step size

const int nmeas = 3; // Number of measurements

// Measurement noise covariance [R]

const double sigmaRpos = 1e-2; // km

const double sigmaRang = (10 / 3600.0) * pi / 180.0;

AlgebraicMatrix<double> R(nmeas, nmeas);

for (int i = 0; i < nmeas; i++) for (int j = 0; j < nmeas; j++) R.at(i, j) = 0.0;

R.at(0, 0) = sigmaRpos * sigmaRpos;

R.at(1, 1) = sigmaRang * sigmaRang;

R.at(2, 2) = sigmaRang * sigmaRang;

// Scaled measurement noise

double sigmaPpos = 1.0;

double sigmaPang = sigmaPpos / a;

AlgebraicMatrix<double> RScaled(nmeas, nmeas);

for (int i = 0; i < nmeas; i++) {

for (int j = 0; j < nmeas; j++) { RScaled.at(i, j) = 0.0; }

}

RScaled.at(0, 0) = R.at(0, 0);

RScaled.at(1, 1) = R.at(1, 1);

RScaled.at(2, 2) = R.at(2, 2);

// **3. Noise Distributions**

std::default_random_engine generator;

std::normal_distribution<double> position_distribution(0.0, 87.88);

std::normal_distribution<double> velocity_distribution(0.0, 6.74e-4);

std::normal_distribution<double> distribution_r(0.0, sigmaRpos);

std::normal_distribution<double> distribution_ang(0.0, sigmaRang);

std::uniform_int_distribution<int> perturbation_dist(0, 19);

// **4. Initial Conditions**

AlgebraicVector<DA> x0(nvar), xc(nvar);

AlgebraicVector<double> x0_double(nvar);

x0[0] = -6045.0; x0[1] = -3490.0; x0[2] = 2500.0;

x0[3] = -3.457;  x0[4] = 6.618;   x0[5] = 2.533;

x0_double = cons(x0);

double sigmaPvel = 1.0;

for (int c = 0; c < 3; c++) xc[c] = x0[c] + sigmaPpos * DA(c + 1);

for (int c = 3; c < 6; c++) xc[c] = x0[c] + sigmaPvel * DA(c + 1);

// **5. Scenario Loop**

const int MC = 20;

const std::string state_vars[nvar] = { "x", "y", "z", "vx", "vy", "vz" };

std::vector<double> azimuth = { 0, 90, 180, 270 };

std::vector<double> elevation = { -90, -45, 0, 45, 90 };

for (int scenario = 0; scenario < 3; scenario++) {

// Set velocity impulse magnitude based on scenario

double V;

if (scenario == 0) V = 0.3;        // Scenario 0: Positive delta_v

else if (scenario == 1) V = 0.0;   // Scenario 1: No delta_v

else V = -0.3;                     // Scenario 2: Negative delta_v

// Compute velocity perturbations

std::vector<Velocity> delta_v_combinations;

for (double theta_deg : azimuth) {

for (double phi_deg : elevation) {

double theta = theta_deg * pi / 180.0;

double phi = phi_deg * pi / 180.0;

Velocity v;

v.vx = V * cos(theta) * cos(phi);

v.vy = V * sin(theta) * cos(phi);

v.vz = V * sin(phi);

delta_v_combinations.push_back(v);

}

}

// Storage Setup

std::vector<std::vector<std::vector<double>>> mc_states(MC, std::vector<std::vector<double>>(nObsPerIter, std::vector<double>(nvar, 0.0)));

std::vector<std::vector<std::vector<double>>> da_states(MC, std::vector<std::vector<double>>(nObsPerIter, std::vector<double>(nvar, 0.0)));

std::vector<int> selected_perturbation(MC, 0);

// Statistics storage

std::vector<std::vector<double>> da_means(nObsPerIter, std::vector<double>(nvar, 0.0));

std::vector<std::vector<double>> da_std_devs(nObsPerIter, std::vector<double>(nvar, 0.0));

// Metrics storage

std::vector<std::vector<double>> err_sum(nvar, std::vector<double>(nObsPerIter, 0.0));

std::vector<std::vector<double>> err_sq_sum(nvar, std::vector<double>(nObsPerIter, 0.0));

std::vector<std::vector<double>> cov_sum(nvar, std::vector<double>(nObsPerIter, 0.0));

// File streams with scenario-specific names

std::ofstream error_files[nvar];

std::ofstream mean_files[nvar];

std::ofstream num_3sigma_files[nvar];

std::ofstream cov_3sigma_files[nvar];

std::string scen_str = "scen" + std::to_string(scenario) + "_";

// Open files

for (int i = 0; i < nvar; i++) {

error_files[i].open(scen_str + state_vars[i] + "_error.csv");

mean_files[i].open(scen_str + state_vars[i] + "_mean.csv");

num_3sigma_files[i].open(scen_str + state_vars[i] + "_num_3sigma.csv");

cov_3sigma_files[i].open(scen_str + state_vars[i] + "_cov_3sigma.csv");

}

// Reset random seed for consistency with original separate runs

generator.seed(100);

// **6. Monte Carlo Loop**

for (int k = 0; k < MC; k++) {

std::cout << "Scenario " << scenario << ", Run Number: " << k + 1 << "/" << MC << std::endl;

int p_rand = perturbation_dist(generator);

selected_perturbation[k] = p_rand;

Velocity delta_v = delta_v_combinations[p_rand];

AlgebraicVector<double> delta_x(nvar);

for (int c = 0; c < 3; c++) delta_x[c] = position_distribution(generator);

for (int c = 3; c < 6; c++) delta_x[c] = velocity_distribution(generator);

AlgebraicVector<double> current_mc = x0_double + delta_x;

AlgebraicVector<DA> xc_current = xc;

AlgebraicMatrix<double> P(nvar, nvar), PScaled(nvar, nvar);

AlgebraicMatrix<double> PZZ(nmeas, nmeas), PXZ(nvar, nmeas);

AlgebraicMatrix<double> INVPZZ(nmeas, nmeas), Kgain(nvar, nmeas);

AlgebraicMatrix<double> PUPD(nvar, nvar);

// Initial covariance matrix [P] set to 2% of state values

for (int i = 0; i < nvar; i++) {

for (int j = 0; j < nvar; j++) {

if (i == j) {

double state_value = std::abs(x0_double[i]);

P.at(i, j) = pow(0.02 * state_value, 2);

}

else {

P.at(i, j) = 0.0;

}

}

}

PScaled = P;

AlgebraicVector<double> mn = x0_double;

double t = 0.0;

for (int obs = 0; obs < nObsPerIter; obs++) {

double tk = t + dtspan;

// Initialize DA variables for current step

for (int i = 0; i < nvar; i++) {

xc_current[i] = mn[i] + DA(i + 1);

}

DA::pushTO(filterorder);

// Propagate states

current_mc = RK78(nvar, current_mc, t, tk, TBP<double>);

xc_current = RK78(nvar, xc_current, t, tk, TBP<DA>);

t = tk;

DA::popTO();

// Apply velocity impulse at observation step 23 (if applicable)

if (obs == 23 && V != 0.0) {

current_mc[3] += delta_v.vx;

current_mc[4] += delta_v.vy;

current_mc[5] += delta_v.vz;

// Note: Impulse not applied to DA part (xc_current)

}

// Store states for analysis

for (int i = 0; i < nvar; i++) {

mc_states[k][obs][i] = current_mc[i];

da_states[k][obs][i] = xc_current[i].cons();

}

DA::pushTO(2 * filterorder);

// Moments computation

DA PscaledPol = 0;

for (int i = 0; i < nvar; i++) {

for (int j = i; j < nvar; j++) {

PscaledPol += PScaled.at(i, j) * DA(i + 1) * DA(j + 1);

}

}

DA moments = computeMoments(PscaledPol);

// Measurements

const auto& st = current_mc;

AlgebraicVector<double> ztrue(nmeas), zpred(nmeas), zdiff(nmeas), mnupdate(nvar);

AlgebraicVector<DA> zf(nmeas);

// Generate true measurements with noise

ztrue[0] = sqrt(st[0] * st[0] + st[1] * st[1] + st[2] * st[2]) + distribution_r(generator);

ztrue[1] = atan2(st[1], st[0]) + distribution_ang(generator);

ztrue[2] = asin(st[2] / ztrue[0]) + distribution_ang(generator);

DA::pushTO(filterorder);

// Predict measurements

AlgebraicVector<DA> xf = xc_current;

zf[0] = sqrt(xf[0] * xf[0] + xf[1] * xf[1] + xf[2] * xf[2]);

zf[1] = atan2(xf[1], xf[0]);

zf[2] = asin(xf[2] / zf[0]);

// Evaluate predicted measurements

zpred[0] = zf[0].evalMonomials(moments);

zpred[1] = zf[1].evalMonomials(moments);

zpred[2] = zf[2].evalMonomials(moments);

// Update mean estimate

for (int i = 0; i < nvar; i++) {

mn[i] = xf[i].evalMonomials(moments);

}

// Center the state and measurement variables

for (int i = 0; i < 3; i++) {

xf[i] = (xf[i] - mn[i]);

xf[i + 3] = (xf[i + 3] - mn[i + 3]);

}

zf[0] = (zf[0] - zpred[0]);

zf[1] = (zf[1] - zpred[1]);

zf[2] = (zf[2] - zpred[2]);

DA::pushTO(2 * filterorder);

// Covariance calculations

AlgebraicMatrix<DA> xx(nvar, nvar), zz(nmeas, nmeas), xz(nvar, nmeas);

// State covariance

for (int i = 0; i < nvar; i++) {

for (int j = i; j < nvar; j++) {

xx.at(i, j) = xf[i] * xf[j];

PUPD.at(i, j) = xx.at(i, j).evalMonomials(moments);

PUPD.at(j, i) = PUPD.at(i, j);

}

}

// Measurement covariance

for (int i = 0; i < nmeas; i++) {

for (int j = i; j < nmeas; j++) {

zz.at(i, j) = zf[i] * zf[j];

PZZ.at(i, j) = zz.at(i, j).evalMonomials(moments);

PZZ.at(j, i) = PZZ.at(i, j);

}

}

PZZ = PZZ + RScaled;

// Cross-covariance

for (int i = 0; i < nvar; i++) {

for (int j = 0; j < nmeas; j++) {

xz.at(i, j) = xf[i] * zf[j];

PXZ.at(i, j) = xz.at(i, j).evalMonomials(moments);

}

}

// Kalman update

PScaled = PUPD;

// Check if PZZ is near-singular and regularize if needed

double det_PZZ = PZZ.det();

if (std::abs(det_PZZ) < 1e-10) {

std::cout << "Warning: PZZ nearly singular, det = " << det_PZZ << std::endl;

for (int i = 0; i < nmeas; i++) PZZ.at(i, i) += 1e-6;

}

INVPZZ = PZZ.inv();

Kgain = PXZ * INVPZZ;

// Update state based on measurement

zdiff = ztrue - zpred;

for (int i = 0; i < nvar; i++) {

mnupdate[i] = 0;

for (int j = 0; j < nmeas; j++) {

mnupdate[i] += Kgain.at(i, j) * zdiff[j];

}

mn[i] = mn[i] + mnupdate[i];

}

// Update covariance

AlgebraicMatrix<double> GG = Kgain * PZZ * Kgain.transpose();

PScaled = PScaled - GG;

// Ensure symmetry in covariance matrix

for (int i = 0; i < nvar; i++) {

for (int j = 0; j < nvar; j++) {

if (i != j) PScaled.at(j, i) = PScaled.at(i, j);

}

}

// Store DA statistics for this run

if (k == 0) {

for (int i = 0; i < nvar; i++) {

da_means[obs][i] = mn[i];

da_std_devs[obs][i] = sqrt(PScaled.at(i, i));

}

}

DA::popTO();

// Compute and Store Metrics

AlgebraicVector<double> corrected_mean = mn;

for (int i = 0; i < nvar; i++) {

double error = current_mc[i] - corrected_mean[i];

err_sum[i][obs] += error;

err_sq_sum[i][obs] += error * error;

error_files[i] << error;

double sigma = 1.0; // sigmaPpos and sigmaPvel are both 1.0

double variance = PScaled.at(i, i) * sigma * sigma;

cov_sum[i][obs] += sqrt(variance);

if (obs < nObsPerIter - 1) {

error_files[i] << ",";

}

else {

error_files[i] << "\n";

}

}

} // End of observation loop

} // End of Monte Carlo loop

// **7. Write Metrics to CSV Files**

for (int obs = 0; obs < nObsPerIter; obs++) {

for (int i = 0; i < nvar; i++) {

// Mean Error

double mean_err = err_sum[i][obs] / MC;

mean_files[i] << mean_err;

// Numerical 3σ (using sample variance)

double sum_err = err_sum[i][obs];

double sum_sq_err = err_sq_sum[i][obs];

double variance = (sum_sq_err - (sum_err * sum_err / MC)) / (MC - 1);

double std_dev = sqrt(variance);

double num_3sigma = 3 * std_dev;

num_3sigma_files[i] << num_3sigma;

// Covariance 3σ

double cov_3sigma = 3 * (cov_sum[i][obs] / MC);

cov_3sigma_files[i] << cov_3sigma;

if (obs < nObsPerIter - 1) {

mean_files[i] << ",";

num_3sigma_files[i] << ",";

cov_3sigma_files[i] << ",";

}

else {

mean_files[i] << "\n";

num_3sigma_files[i] << "\n";

cov_3sigma_files[i] << "\n";

}

}

}

// **8. Clean Up**

for (int i = 0; i < nvar; i++) {

error_files[i].close();

mean_files[i].close();

num_3sigma_files[i].close();

cov_3sigma_files[i].close();

}

std::cout << "Scenario " << scenario << " complete. Data written to CSV files.\n";

} // End of scenario loop

std::cout << "Simulation complete for all scenarios.\n";

return 0;

}