function main

% Constants

mu = 398600; % km^3/s^2

% Initial conditions

x0 = [-1871.0; 10639.0; 532.0; -6.2506; -1.0035; 0.1182];

delta_x = [-0.1871; 1.0639; 0.0532; -0.062506; -0.10035; 0.01182];

% Compute the orbital period

a = 12000; % semi-major axis (in km)

T = 2 * pi * sqrt(a^3 / mu); % Period in seconds

% Final state using RKF78

tol = 1e-1; % tolerance

xf = RKF78(@TBP, x0, 0, T, mu, tol);

% Output initial and final conditions

fprintf('Initial conditions (x0):\n');

print_vector(x0);

fprintf('Final conditions (xf) rounded to 6 digits:\n');

print_vector(round(xf, 6));

% Displaced initial conditions

x0_displaced = x0 + delta_x;

fprintf('Displaced Initial condition (x0 + delta_x):\n');

print_vector(round(x0_displaced, 6));

% Displaced final conditions

xf_displaced = RKF78(@TBP, x0_displaced, 0, T, mu, tol);

fprintf('Displaced Final condition (xf evaluated at delta_x) rounded to 6 digits:\n');

print_vector(round(xf_displaced, 6));

end

function dx = TBP(~, x, mu)

% Two-body problem (TBP) function

pos = x(1:3); % position vector

vel = x(4:6); % velocity vector

r = norm(pos); % magnitude of position vector

% State derivatives

dx = zeros(6,1);

dx(1:3) = vel; % velocity components

dx(4) = -mu * pos(1) / (r^3); % acceleration in x direction

dx(5) = -mu * pos(2) / (r^3); % acceleration in y direction

dx(6) = -mu * pos(3) / (r^3); % acceleration in z direction

end

function xf = RKF78(TBP, x0, t0, t1, mu, tol)

% Runge-Kutta-Fehlberg 7/8 method with adaptive step size

hmax = 0.1; % maximum step size

hmin = 1e-2; % minimum step size

h = hmax; % initial step size

t = t0;

x = x0;

while t < t1

if t + h > t1

h = t1 - t; % adjust the final step size

end

% Compute the 7 stages for RKF78

k1 = h * feval(TBP, t, x, mu);

k2 = h * feval(TBP, t + h * (1.0 / 9.0), x + (1.0 / 9.0) * k1, mu);

k3 = h * feval(TBP, t + h * (1.0 / 6.0), x + (1.0 / 12.0) * k1 + (1.0 / 4.0) * k2, mu);

k4 = h * feval(TBP, t + h * (1.0 / 3.0), x + (1.0 / 8.0) * k1 + (3.0 / 8.0) * k3, mu);

k5 = h * feval(TBP, t + h * (1.0 / 2.0), x + (148.0 / 1331.0) * k1 + (150.0 / 1331.0) * k3 + (56.0 / 1331.0) * k4, mu);

k6 = h * feval(TBP, t + h * (2.0 / 3.0), x + (739.0 / 9064.0) * k1 + (181.0 / 530.0) * k3 + (289.0 / 2108.0) * k4 + (215.0 / 828.0) * k5, mu);

k7 = h * feval(TBP, t + h * (5.0 / 6.0), x + (64.0 / 535.0) * k1 + (31.0 / 70.0) * k3 + (6.0 / 35.0) * k5 + (6.0 / 175.0) * k6, mu);

% Calculate final result using weighted sum of ks

x_new = x + (41.0 / 840.0) * k1 + (34.0 / 105.0) * k3 + (9.0 / 35.0) * k5 + (9.0 / 280.0) * k7;

% Error estimate

err = h * ((41.0 / 840.0) * k1 - (9.0 / 35.0) * k5 + (9.0 / 280.0) * k7);

err_norm = norm(err); % error norm

if err_norm > tol

% Reduce step size

h = h * 0.9 * (tol / err_norm)^(1/5);

if h < hmin

h = hmin;

end

else

% Accept the step

x = x_new;

t = t + h;

% Increase step size

h = h * 0.9 * (tol / err_norm)^(1/5);

if h > hmax

h = hmax;

end

end

end

xf = x;

end

function print_vector(vec)

% Helper function to print vectors rounded to 6 digits

for i = 1:length(vec)

fprintf('%12.6f\n', round(vec(i), 6)); % Round to 6 digits

end

end