function RKF78_OrbitalDynamics

% Constants

mu = 398600; % Gravitational parameter, km^3/s^2

a = 8788;   % Semi-major axis in km

% Initial conditions

x0 = [-1871; 10639; 532; -6.2506; -1.0035; 0.1182]; % Initial state vector

% Time span (half orbital period)

T = 2 * pi * sqrt(a^3 / mu); % Period in seconds

t0 = 0;

% Tolerance for RKF78

tol = 1e-6; % Adjusted tolerance for better accuracy

h_initial = 10; % Initial step size

% Run the RKF78 integrator and get trajectory

[xf, trajectory] = RKF78(@TwoBodyProblem, x0, t0, h_initial, T, tol);

% Display results

fprintf('\nInitial conditions:\n');

disp(x0);

fprintf('\nFinal conditions (up to 6 digits):\n');

disp(round(xf, 6)); % Display final conditions up to 6 digits

% Calculate orbital elements for x0 and xf

% elements0 = stateToOrbitalElements(x0, mu);

% elementsf = stateToOrbitalElements(xf, mu);

end

% Two-body problem function

function dxdt = TwoBodyProblem(~, x)

mu = 398600; % Gravitational parameter, km^3/s^2

r = norm(x(1:3)); % Magnitude of position vector

% Derivatives of the state vector

dxdt = zeros(6,1);

dxdt(1:3) = x(4:6); % Velocity components

dxdt(4) = -mu * x(1) / r^3; % Acceleration in x direction

dxdt(5) = -mu * x(2) / r^3; % Acceleration in y direction

dxdt(6) = -mu * x(3) / r^3; % Acceleration in z direction

end

% Runge-Kutta-Fehlberg 7/8 method

function [y, out, h_next] = RKF78(func, y0, x, h, xmax, tolerance)

% Initialize variables

ATTEMPTS = 12;

MIN_SCALE_FACTOR = 0.125;

MAX_SCALE_FACTOR = 4.0;

err_exponent = 1.0 / 7.0;

% Ensure that the step size is positive and valid

if (xmax < x || h <= 0.0)

y = y0;

out = -2;

h_next = h;

return

end

h_next = h;

y = y0;

% Ensure that the step size does not exceed the integration range

if (h > (xmax - x))

h = xmax - x;

end

% Adjust tolerance per unit length of integration interval

tolerance = tolerance / (xmax - x);

% Integrate the differential equation y' = f(x, y)

while (x < xmax)

scale = 1.0;

for i = 1:ATTEMPTS

[temp_y, err] = Runge_Kutta(func, y0, x, h);

err = norm(err);

if (err == 0.0)

scale = MAX_SCALE_FACTOR;

break

end

scale = 0.8 * (tolerance / err)^(err_exponent);

scale = min(max(scale, MIN_SCALE_FACTOR), MAX_SCALE_FACTOR);

if (err < tolerance)

break

end

h = h * scale;

if (x + h > xmax)

h = xmax - x;

elseif (x + h + 0.5 * h > xmax)

h = 0.5 * h;

end

end

if (i >= ATTEMPTS)

h_next = h * scale;

out = -1;

return

end

y0 = temp_y;

x = x + h;

h_next = h;

if (x + h > xmax)

h = xmax - x;

elseif (x + h + 0.5 * h > xmax)

h = 0.5 * h;

end

end

y = temp_y;

out = 0;

end

function [y, out] = Runge_Kutta(func, y, x0, h)

c_1_11 = 41.0 / 840.0;

c6 = 34.0 / 105.0;

c_7_8= 9.0 / 35.0;

c_9_10 = 9.0 / 280.0;

a2 = 2.0 / 27.0;

a3 = 1.0 / 9.0;

a4 = 1.0 / 6.0;

a5 = 5.0 / 12.0;

a6 = 1.0 / 2.0;

a7 = 5.0 / 6.0;

a8 = 1.0 / 6.0;

a9 = 2.0 / 3.0;

a10 = 1.0 / 3.0;

b31 = 1.0 / 36.0;

b32 = 3.0 / 36.0;

b41 = 1.0 / 24.0;

b43 = 3.0 / 24.0;

b51 = 20.0 / 48.0;

b53 = -75.0 / 48.0;

b54 = 75.0 / 48.0;

b61 = 1.0 / 20.0;

b64 = 5.0 / 20.0;

b65 = 4.0 / 20.0;

b71 = -25.0 / 108.0;

b74 =  125.0 / 108.0;

b75 = -260.0 / 108.0;

b76 =  250.0 / 108.0;

b81 = 31.0/300.0;

b85 = 61.0/225.0;

b86 = -2.0/9.0;

b87 = 13.0/900.0;

b91 = 2.0;

b94 = -53.0/6.0;

b95 = 704.0 / 45.0;

b96 = -107.0 / 9.0;

b97 = 67.0 / 90.0;

b98 = 3.0;

b10_1 = -91.0 / 108.0;

b10_4 = 23.0 / 108.0;

b10_5 = -976.0 / 135.0;

b10_6 = 311.0 / 54.0;

b10_7 = -19.0 / 60.0;

b10_8 = 17.0 / 6.0;

b10_9 = -1.0 / 12.0;

b11_1 = 2383.0 / 4100.0;

b11_4 = -341.0 / 164.0;

b11_5 = 4496.0 / 1025.0;

b11_6 = -301.0 / 82.0;

b11_7 = 2133.0 / 4100.0;

b11_8 = 45.0 / 82.0;

b11_9 = 45.0 / 164.0;

b11_10 = 18.0 / 41.0;

b12_1 = 3.0 / 205.0;

b12_6 = - 6.0 / 41.0;

b12_7 = - 3.0 / 205.0;

b12_8 = - 3.0 / 41.0;

b12_9 = 3.0 / 41.0;

b12_10 = 6.0 / 41.0;

b13_1 = -1777.0 / 4100.0;

b13_4 = -341.0 / 164.0;

b13_5 = 4496.0 / 1025.0;

b13_6 = -289.0 / 82.0;

b13_7 = 2193.0 / 4100.0;

b13_8 = 51.0 / 82.0;

b13_9 = 33.0 / 164.0;

err_factor  = -41.0 / 840.0;

h2_7 = a2 * h;b13_10 = 12.0 / 41.0;

k1 = func(x0, y);

% Compute k2 to k13 similar to your original code...

k1 = func(x0, y);

k2 = func(x0+h2_7, y + h2_7 * k1);

k3 = func(x0+a3*h, y + h * (b31*k1 + b32*k2));

k4 = func(x0+a4*h, y + h * (b41*k1 + b43*k3));

k5 = func(x0+a5*h, y + h * (b51*k1 + b53*k3 + b54*k4));

k6 = func(x0+a6*h, y + h * (b61*k1 + b64*k4 + b65*k5));

k7 = func(x0+a7*h, y + h * (b71*k1 + b74*k4 + b75*k5 + b76*k6));

k8 = func(x0+a8*h, y + h * (b81*k1 + b85*k5 + b86*k6 + b87*k7));

k9 = func(x0+a9*h, y + h * (b91*k1 + b94*k4 + b95*k5 + b96*k6 + b97*k7 + b98*k8));

k10 = func(x0+a10*h, y + h * (b10_1*k1 + b10_4*k4 + b10_5*k5 + b10_6*k6...

+ b10_7*k7 + b10_8*k8 + b10_9*k9));

k11 = func(x0+h, y + h * (b11_1*k1 + b11_4*k4 + b11_5*k5 + b11_6*k6...

+ b11_7*k7 + b11_8*k8 + b11_9*k9 + b11_10 * k10));

k12 = func(x0, y + h * (b12_1*k1 + b12_6*k6 + b12_7*k7 + b12_8*k8...

+ b12_9*k9 + b12_10 * k10));

k13 = func(x0+h, y + h * (b13_1*k1 + b13_4*k4 + b13_5*k5 + b13_6*k6...

+ b13_7*k7 + b13_8*k8 + b13_9*k9 + b13_10*k10 + k12));

% Calculate the final result

y = y + h * (c_1_11 * (k1 + k11) + c6 * k6 + c_7_8 * (k7 + k8) + c_9_10 * (k9 + k10));

out = err_factor * (k1 + k11 - k12 - k13);

end