% Clear workspace and figures

clear all;

close all;

% Define parameters

MC = 7;             % Number of Monte Carlo runs (matched to C++ code)

nObs = 24;          % Number of observations per run

state_vars = {'x', 'y', 'z', 'Vx', 'Vy', 'Vz'};

n_var = length(state_vars);

mu = 398600.0;      % Gravitational parameter (km^3/s^2)

a = 8788.0;         % Semi-major axis (km)

T = 2 * pi * sqrt((a^3) / mu); % Orbital period (s)

dt = T / nObs;      % Time step size

t_seconds = 0:dt:T-dt;      % Time vector (1x24) in seconds

t = t_seconds / T;  % Normalized time vector from 0 to 1

%% --- Plot Errors, Mean, Num 3-Sigma, Cov 3-Sigma, and P_final 3-Sigma ---

% File names for all metrics

error_files = {'error_x.csv', 'error_y.csv', 'error_z.csv', ...

'error_Vx.csv', 'error_Vy.csv', 'error_Vz.csv'};

mean_files = {'mean_x.csv', 'mean_y.csv', 'mean_z.csv', ...

'mean_Vx.csv', 'mean_Vy.csv', 'mean_Vz.csv'};

num_3sigma_files = {'num_3sigma_x.csv', 'num_3sigma_y.csv', 'num_3sigma_z.csv', ...

'num_3sigma_Vx.csv', 'num_3sigma_Vy.csv', 'num_3sigma_Vz.csv'};

cov_3sigma_files = {'cov_3sigma_x.csv', 'cov_3sigma_y.csv', 'cov_3sigma_z.csv', ...

'cov_3sigma_Vx.csv', 'cov_3sigma_Vy.csv', 'cov_3sigma_Vz.csv'};

P_final_3sigma_files = {'P_final_3sigma_x.csv', 'P_final_3sigma_y.csv', 'P_final_3sigma_z.csv', ...

'P_final_3sigma_Vx.csv', 'P_final_3sigma_Vy.csv', 'P_final_3sigma_Vz.csv'};

% Titles and units for subplots

titles = {'Error in x', 'Error in y', 'Error in z', ...

'Error in Vx', 'Error in Vy', 'Error in Vz'};

units = {'km', 'km', 'km', 'km/s', 'km/s', 'km/s'};

% Create a single figure with subplots for errors

figure('Name', 'Error Analysis Across State Variables', 'NumberTitle', 'off');

for i = 1:n_var

% Read CSV files

error_data = readmatrix(error_files{i});        % 7x24 (MC runs x observations)

mean_row = readmatrix(mean_files{i});           % 1x24

num_3sigma_row = readmatrix(num_3sigma_files{i}); % 1x24

cov_3sigma_row = readmatrix(cov_3sigma_files{i}); % 1x24

P_final_3sigma_data = readmatrix(P_final_3sigma_files{i}); % 7x24

% Verify sizes

if size(error_data, 1) ~= MC || size(error_data, 2) ~= nObs

error('Error data size mismatch for %s: expected %dx%d, got %dx%d', ...

error_files{i}, MC, nObs, size(error_data, 1), size(error_data, 2));

end

if length(mean_row) ~= nObs || length(num_3sigma_row) ~= nObs || length(cov_3sigma_row) ~= nObs

error('Metric data size mismatch for %s: expected length %d', ...

state_vars{i}, nObs);

end

if size(P_final_3sigma_data, 1) ~= MC || size(P_final_3sigma_data, 2) ~= nObs

error('P_final_3sigma data size mismatch for %s: expected %dx%d, got %dx%d', ...

P_final_3sigma_files{i}, MC, nObs, size(P_final_3sigma_data, 1), size(P_final_3sigma_data, 2));

end

% Compute mean of P_final_3sigma across MC runs (1x24)

P_final_3sigma_row = mean(P_final_3sigma_data, 1);

% Subplot arrangement: 2 rows, 3 columns

subplot(2, 3, i);

hold on;

% Plot all MC error runs in gray (no legend entry)

for run = 1:MC

plot(t, error_data(run, :), 'Color', [0.7 0.7 0.7], 'LineWidth', 0.1, 'HandleVisibility', 'off');

end

% Plot mean error in blue

plot(t, mean_row, 'Color', 'b', 'LineWidth', 1, 'DisplayName', 'Mean Error');

% Plot numerical ±3σ in green

plot(t, num_3sigma_row, 'Color', 'g', 'LineWidth', 0.5, 'DisplayName', '+3σ Numerical');

plot(t, -num_3sigma_row, 'Color', 'g', 'LineWidth', 0.5, 'DisplayName', '-3σ Numerical');

% Plot covariance ±3σ in red dashed lines

plot(t, cov_3sigma_row, 'Color', 'r', 'LineStyle', '--', 'LineWidth', 0.5, 'DisplayName', '+3σ Covariance');

plot(t, -cov_3sigma_row, 'Color', 'r', 'LineStyle', '--', 'LineWidth', 0.5, 'DisplayName', '-3σ Covariance');

% Plot P_final ±3σ in magenta dashed lines (mean across MC runs)

plot(t, P_final_3sigma_row, 'Color', 'm', 'LineStyle', ':', 'LineWidth', 0.5, 'DisplayName', '+3σ P_final');

plot(t, -P_final_3sigma_row, 'Color', 'm', 'LineStyle', ':', 'LineWidth', 0.5, 'DisplayName', '-3σ P_final');

% Add vertical line at T/2 (which is 0.5 in normalized time)

plot([0.5 0.5], ylim, 'b--', 'LineWidth', 1, 'DisplayName', 'T/2');

% Labels and title

xlabel('Time (normalized)');

ylabel(['Error (' units{i} ')']);

title(titles{i});

grid on;

% Show legend for labeled lines only

legend('show', 'Location', 'best');

hold off;

end

% Adjust figure layout

set(gcf, 'Position', [500, 200, 800, 600]);

sgtitle('Error vs Time with 3σ Metrics (7 MC x 24 Obs)');

%% --- Plot Weight of True Model for Each MC Run ---

% Read model_selection.csv

selection_data = readtable('model_selection.csv');

% Columns: MC_run, selected_model, impulse_vx, impulse_vy, impulse_vz

% Read model_weights.csv

weights_data = readtable('model_weights.csv');

% Columns: MC_run, time, model1_weight, model2_weight, ..., model246_weight

% Get unique MC runs

mc_runs = unique(selection_data.MC_run);

n_mc = length(mc_runs);

% Create a new figure for model weights

figure('Name', 'Weight of True Model Across Monte Carlo Runs', 'NumberTitle', 'off');

hold on;

% Colors for each MC run

colors = lines(n_mc);

% Plot weight of true model for each MC run

for k = 1:n_mc

mc_run = mc_runs(k);

% Get selected model for this MC run

selected_model = selection_data.selected_model(selection_data.MC_run == mc_run);

if length(selected_model) > 1

error('Multiple selected models for MC run %d', mc_run);

end

selected_model = selected_model(1);

% Get weights for this MC run

idx = weights_data.MC_run == mc_run;

times = weights_data.time(idx);

% Normalize times to 0-1 scale

times_normalized = times / T;

% Get the weight column for the selected model

weight_col = ['model' num2str(selected_model) '_weight'];

if ~ismember(weight_col, weights_data.Properties.VariableNames)

error('Weight column %s not found', weight_col);

end

weights_true = weights_data.(weight_col)(idx);

% Plot

plot(times_normalized, weights_true, 'Color', colors(k, :), 'LineWidth', 1, ...

'DisplayName', sprintf('MC Run %d (Model %d)', mc_run, selected_model));

end

% Add vertical line at T/2 (which is 0.5 in normalized time)

plot([0.5 0.5], ylim, 'b--', 'LineWidth', 1, 'DisplayName', 'T/2');

% Labels and title

xlabel('Time (normalized)');

ylabel('Weight of True Model');

title('Weight of True Model Over Time for Each Monte Carlo Run');

legend('show', 'Location', 'best');

grid on;

% Adjust figure layout

set(gcf, 'Position', [500, 200, 800, 400]);

hold off;