#include <dace/dace.h>

#include <cmath>

#include <iostream>

#include <fstream>

#include <random>  // for random numbers

using namespace std;

using namespace DACE;

// Random number generator for normal distribution

std::default_random_engine generator;

std::normal_distribution<double> distribution(0.0, .5);  // mean = 0, stddev = .5

// Two-body problem (TBP) function

template<typename T> AlgebraicVector<T> TBP(AlgebraicVector<T> x, double t) {

AlgebraicVector<T> pos(3), res(6);

pos[0] = x[0];

pos[1] = x[1];

pos[2] = x[2];

T r = pos.vnorm();  // magnitude of position vector

const double mu = 398600;  // gravitational parameter, km^3/s^2

res[0] = x[3];  // velocity in x direction

res[1] = x[4];  // velocity in y direction

res[2] = x[5];  // velocity in z direction

// gravitational acceleration

res[3] = -mu * pos[0] / (r * r * r);

res[4] = -mu * pos[1] / (r * r * r);

res[5] = -mu * pos[2] / (r * r * r);

return res;

}

// Runge-Kutta-Fehlberg 7/8 method (RKF78)

template<typename T> AlgebraicVector<T> RKF78(AlgebraicVector<T> x, double t0, double t1, double tol = 1e-1) {

const double hmax = 0.1;

const double hmin = 1e-2;

double h = hmax;

double t = t0;

// Initialize the next state

AlgebraicVector<T> x_next(6);

// Runge-Kutta loop to propagate over the time interval

while (t < t1) {

if (t + h > t1)

h = t1 - t;  // Adjust final step size

// Compute the stages for the RKF78 method (7 stages)

AlgebraicVector<T> k1 = TBP(x, t);

AlgebraicVector<T> k2 = TBP(x + h * (1.0 / 9.0) * k1, t + h * (1.0 / 9.0));

AlgebraicVector<T> k3 = TBP(x + h * (1.0 / 12.0) * k1 + h * (1.0 / 4.0) * k2, t + h * (1.0 / 6.0));

AlgebraicVector<T> k4 = TBP(x + h * (1.0 / 8.0) * k1 + h * (3.0 / 8.0) * k3, t + h * (1.0 / 3.0));

AlgebraicVector<T> k5 = TBP(x + h * (148.0 / 1331.0) * k1 + h * (150.0 / 1331.0) * k3 + h * (56.0 / 1331.0) * k4, t + h * (1.0 / 2.0));

AlgebraicVector<T> k6 = TBP(x + h * (739.0 / 9064.0) * k1 + h * (181.0 / 530.0) * k3 + h * (289.0 / 2108.0) * k4 + h * (215.0 / 828.0) * k5, t + h * (2.0 / 3.0));

AlgebraicVector<T> k7 = TBP(x + h * (64.0 / 535.0) * k1 + h * (31.0 / 70.0) * k3 + h * (6.0 / 35.0) * k5 + h * (6.0 / 175.0) * k6, t + h * (5.0 / 6.0));

// Calculate the next state using the weighted sum of ks

x_next = x + h * (41.0 / 840.0) * k1 + h * (34.0 / 105.0) * k3 + h * (9.0 / 35.0) * k5 + h * (9.0 / 280.0) * k7;

// Error estimate

AlgebraicVector<T> err = h * ((41.0 / 840.0) * k1 - (9.0 / 35.0) * k5 + (9.0 / 280.0) * k7);

// Compute the error norm (as a double)

double err_norm = err.cons().vnorm();

// Adjust the step size based on the error

if (err_norm > tol) {

// Reduce step size

h = h * 0.9 * pow(tol / err_norm, 0.2);

if (h < hmin) h = hmin;

}

else {

// Accept the step

x = x_next;

t += h;

// Increase step size for next iteration

h = h * 0.9 * pow(tol / err_norm, 0.2);

if (h > hmax) h = hmax;

}

}

// Return the final propagated state

return x_next;

}

int main(void) {

// Initialize DACE

DA::init(3, 6);

// Declare initial conditions

AlgebraicVector<DA> x0(6), xf(6);

AlgebraicVector<double> x0_double(6), delta_x(6), xf_delta(6);

// Set initial conditions for x0

x0[0] = -1871.0 + DA(1); x0[1] = 10639.0 + DA(2); x0[2] = 532.0 + DA(3);

x0[3] = -6.2506 + DA(4); x0[4] = -1.0035 + DA(5); x0[5] = 0.1182 + DA(6);

// Convert x0 to double for RKF78

x0_double[0] = -1871.0; x0_double[1] = 10639.0; x0_double[2] = 532.0;

x0_double[3] = -6.2506; x0_double[4] = -1.0035; x0_double[5] = 0.1182;

// Define orbital period T

const double mu = 398600;

const double pi = 4.0 * atan(1.0);

double a = 12000;  // semi-major axis

double T = 2 * pi * sqrt(a * a * a / mu);  // Period in seconds

// Open file to save results

ofstream outfile("x1_x2_data.txt");

// Propagate x1 using Taylor expansion (xf evaluated at delta_x)

xf = RKF78(x0, 0.0, T);

// Loop over 100 iterations

for (int i = 0; i < 100; ++i) {

// Reset delta_x for each iteration

delta_x[0] = 0;

delta_x[1] = 0;

delta_x[2] = 0;

// Random velocity perturbations with 0.5 km/s standard deviation

delta_x[3] = 0.5 * distribution(generator);  // perturbation in Vx

delta_x[4] = 0.5 * distribution(generator);  // perturbation in Vy

delta_x[5] = 0.5 * distribution(generator);  // perturbation in Vz

// Propagate x1 using RKF78 with the perturbed initial condition

AlgebraicVector<double> g = x0_double + delta_x;

xf_delta = RKF78(g, 0.0, T);

AlgebraicVector<double> X1 = xf.eval(delta_x);

// Write x1 and x2 to file

outfile << "Iteration " << i + 1 << ":\n";

outfile << "X2 (Displaced final state): " << xf_delta << "\n";

outfile << "X1 (Taylor-expanded final state): " << X1 << "\n\n";

}

outfile.close();

cout << "Results saved to x1_x2_data.txt" << endl;

return 0;

}