int main() {

// **1. Orbital Parameters**

const double mu = 398600.0;

const double pi = 3.14159265358979323846;

const double a = 8788.0;

const double T = 2.0 * pi * sqrt(pow(a, 3) / mu);

const int nObsPerIter = 24;

const double dtspan = T / nObsPerIter;

const int nvar = 6;

const int MC = 1000;  // Monte Carlo runs

// **2. Noise Distributions**

std::default_random_engine generator;

std::normal_distribution<double> position_distribution(0.0, 87.88);

std::normal_distribution<double> velocity_distribution(0.0, 6.74e-4);

std::uniform_real_distribution<double> V_dist(0.0, 0.3);

std::uniform_real_distribution<double> theta_dist(0.0, 359.0);

std::uniform_real_distribution<double> phi_dist(-90.0, 90.0);

// **3. Initial Conditions**

AlgebraicVector<double> x0_double(nvar);

x0_double[0] = -6045.00156; x0_double[1] = -3490.97844; x0_double[2] = 2500.01024;

x0_double[3] = -3.457248406; x0_double[4] = 6.618417173; x0_double[5] = 2.533178193;

// **4. Accumulators for Statistics**

std::vector<std::vector<double>> sum_x(nObsPerIter, std::vector<double>(nvar, 0.0));

std::vector<std::vector<double>> sum_xx(nObsPerIter, std::vector<double>(nvar, 0.0));

// CHANGE: Add storage for final states and trajectories

std::vector<std::vector<double>> final_states(MC, std::vector<double>(nvar)); // MC runs x state components

std::vector<std::vector<double>> trajectories(nvar, std::vector<double>(MC * nObsPerIter)); // Component x (MC * nObsPerIter)

// **5. Main Monte Carlo Loop**

for (int k = 0; k < MC; k++) {

if (k % 100 == 0)

std::cout << "Run Number: " << k + 1 << "/" << MC << std::endl;

// **5.1 Generate perturbed initial state**

AlgebraicVector<double> current_mc = x0_double;

for (int c = 0; c < 3; c++)

current_mc[c] += position_distribution(generator);

for (int c = 3; c < 6; c++)

current_mc[c] += velocity_distribution(generator);

// **5.2 Generate random impulse**

double V_truth = V_dist(generator);

double theta_truth = theta_dist(generator);

double phi_truth = phi_dist(generator);

double theta_rad = theta_truth * pi / 180.0;

double phi_rad = phi_truth * pi / 180.0;

double vx_imp = V_truth * cos(theta_rad) * cos(phi_rad);

double vy_imp = V_truth * sin(theta_rad) * cos(phi_rad);

double vz_imp = V_truth * sin(phi_rad);

// **5.3 Propagate state through all observation intervals**

double t = 0.0;

for (int obs = 0; obs < nObsPerIter; obs++) {

double tk = t + dtspan;

current_mc = RK78(nvar, current_mc, t, tk, TBP<double>);

// Apply impulse at midpoint

if (obs == nObsPerIter / 2) {

current_mc[3] += vx_imp;

current_mc[4] += vy_imp;

current_mc[5] += vz_imp;

}

t = tk;

// **5.4 Accumulate statistics**

for (int j = 0; j < nvar; j++) {

double val = current_mc[j];

sum_x[obs][j] += val;

sum_xx[obs][j] += val * val;

}

// CHANGE: Store trajectory data

for (int j = 0; j < nvar; j++) {

trajectories[j][k * nObsPerIter + obs] = current_mc[j];

}

// CHANGE: Store final state at last observation

if (obs == nObsPerIter - 1) {

for (int j = 0; j < nvar; j++) {

final_states[k][j] = current_mc[j];

}

}

}

}

// **6. Compute Statistics**

std::vector<std::vector<double>> mean(nObsPerIter, std::vector<double>(nvar, 0.0));

std::vector<std::vector<double>> variance(nObsPerIter, std::vector<double>(nvar, 0.0));

std::vector<std::vector<double>> bounds_3sigma(nObsPerIter, std::vector<double>(nvar, 0.0));

for (int obs = 0; obs < nObsPerIter; obs++) {

for (int j = 0; j < nvar; j++) {

mean[obs][j] = sum_x[obs][j] / MC;

variance[obs][j] = (sum_xx[obs][j] - (sum_x[obs][j] * sum_x[obs][j]) / MC) / (MC - 1);

if (variance[obs][j] < 0) variance[obs][j] = 0;

bounds_3sigma[obs][j] = 3.0 * sqrt(variance[obs][j]);

}

}

// **7. Write Results**

std::string components[6] = { "x", "y", "z", "vx", "vy", "vz" };

for (int i = 0; i < 6; i++) {

std::ofstream mean_file("mean_" + components[i] + ".csv");

std::ofstream var_file("var_" + components[i] + ".csv");

std::ofstream bound_file("bounds_3sigma_" + components[i] + ".csv");

mean_file << std::scientific << std::setprecision(15);

var_file << std::scientific << std::setprecision(15);

bound_file << std::scientific << std::setprecision(15);

for (int obs = 0; obs < nObsPerIter; obs++) {

mean_file << mean[obs][i];

var_file << variance[obs][i];

bound_file << bounds_3sigma[obs][i];

if (obs < nObsPerIter - 1) {

mean_file << ",";

var_file << ",";

bound_file << ",";

}

}

mean_file << "\n";

var_file << "\n";

bound_file << "\n";

}

// CHANGE: Write final states to file

std::ofstream final_file("final_states.csv");

final_file << std::scientific << std::setprecision(15);

for (int k = 0; k < MC; k++) {

for (int j = 0; j < nvar; j++) {

final_file << final_states[k][j];

if (j < nvar - 1) final_file << ",";

}

final_file << "\n";

}

final_file.close();

// CHANGE: Write trajectories to separate files

std::string traj_files[6] = { "traj_x.csv", "traj_y.csv", "traj_z.csv", "traj_vx.csv", "traj_vy.csv", "traj_vz.csv" };

for (int j = 0; j < nvar; j++) {

std::ofstream traj_file(traj_files[j]);

traj_file << std::scientific << std::setprecision(15);

for (int k = 0; k < MC; k++) {

for (int obs = 0; obs < nObsPerIter; obs++) {

traj_file << trajectories[j][k * nObsPerIter + obs];

if (obs < nObsPerIter - 1) traj_file << ",";

}

traj_file << "\n";

}

traj_file.close();

}

// CHANGE: Write dtspan to file

std::ofstream dt_file("dtspan.txt");

dt_file << std::scientific << std::setprecision(15) << dtspan << "\n";

dt_file.close();

std::cout << "Propagation complete. Data written to files.\n";

return 0;

}