#include <dace/dace.h>

#include <cmath>

#include <iostream>

using namespace std;

using namespace DACE;

//// Two-body problem (TBP) function

template<typename T> AlgebraicVector<T> TBP(AlgebraicVector<T> x, double t)

{

AlgebraicVector<T> pos(3), res(6);

pos[0] = x[0];

pos[1] = x[1];

pos[2] = x[2];

T r = pos.vnorm(); // used for finding magnitude

const double mu = 398600; // km^3/s^2

res[0] = x[3]; // velocity in x direction

res[1] = x[4]; // velocity in y direction

res[2] = x[5]; // velocity in z direction

// acceleration due to gravity

res[3] = -mu * pos[0] / (r * r * r);

res[4] = -mu * pos[1] / (r * r * r);

res[5] = -mu * pos[2] / (r * r * r);

return res;

}

// Runge-Kutta 4th Order method

template<typename T> AlgebraicVector<T> RK4(AlgebraicVector<T> x, double t0, double t1)

{

const double hmax = 0.1;

int steps = ceil((t1 - t0) / hmax);

double h = (t1 - t0) / steps;

double t = t0;

for (int i = 0; i < steps;

i++)

{

// Compute the 4 Runge-Kutta stages

AlgebraicVector<T> k1 = TBP(x, t);

AlgebraicVector<T> k2 = TBP(x + 0.5 * h * k1, t + 0.5 * h);

AlgebraicVector<T> k3 = TBP(x + 0.5 * h * k2, t + 0.5 * h);

AlgebraicVector<T> k4 = TBP(x + h * k3, t + h);

// Update x for the next step using RK4 formula

x = x + (h / 6.0) * (k1 + 2.0 * k2 + 2.0 * k3 + k4);

// Increment time

t += h;

}

return x;

}

int main(void)

{

// Initialize DACE (Differential Algebra Core Engine)

DA::init(3, 6);

// Declare initial and final state vectors

AlgebraicVector<DA> x0(6), xf(6), delta_xf(6), y10(6), y1f(6);

AlgebraicVector<double> delta_x(6);

// Set initial conditions

const double mu = 398600;

const double ecc = 0.1;

// Initial position and velocity in DA form for x0 and delta_x

x0[0] = -1871.0 + DA(1); x0[1] = 10639.0 + DA(2); x0[2] = 532.0 + DA(3);

x0[3] = -6.2506 + DA(4); x0[4] = -1.0035 + DA(5); x0[5] = 0.1182 + DA(6);

delta_x[0] = -.1871; delta_x[1] = 1.0639; delta_x[2] = .0532;

delta_x[3] = -.062506; delta_x[4] = -.10035; delta_x[5] = 0.01182;

// Compute the orbital period

const double pi = 4.0 * atan(1.0);

double a = 12000; // semi-major axis

double T = 2 * pi * sqrt(a * a * a / mu); // Period in seconds

// Final state using RKF78 for x0 and delta_x

xf = RK4(x0, 0.0, T);

cout << endl << "Initil  conditions (xf):" << endl;

cout << x0.cons() << endl;

cout << endl << "Final conditions (xf):" << endl;

cout << xf.cons() << endl;

// for y1=x0+delta_x

cout << "Displaced Initial condition:" << endl;

cout << x0.cons() + delta_x << endl;

cout << endl << "Displaced Final condition:" << endl;

cout << xf.eval(delta_x) << endl;

return 0;

}