int main() {

struct Velocity {

double vx, vy, vz;

};

// **1. Initialize DACE**

DA::setEps(0.0);

const int filterorder = 2;

const int nvar = 6;

DA::init(2 * filterorder, nvar);

// **2. Orbital Parameters**

const double mu = 398600.0; // Gravitational parameter (km^3/s^2)

const double pi = 3.14159265358979323846;

const double a = 8788.0; // Semi-major axis (km)

const double T = 2.0 * pi * sqrt(pow(a, 3) / mu); // Orbital period (s)

const int nObsPerIter = 96; // Number of observation steps

const double dtspan = T / nObsPerIter; // Time step size

const int nmeas = 3; // Number of measurements

// Measurement noise covariance [R]

const double sigmaRpos = 1e-1; // km

const double sigmaRang = (100 / 3600.0) * pi / 180.0;

AlgebraicMatrix<double> R(nmeas, nmeas);

for (int i = 0; i < nmeas; i++) for (int j = 0; j < nmeas; j++) R.at(i, j) = 0.0;

R.at(0, 0) = sigmaRpos * sigmaRpos;

R.at(1, 1) = sigmaRang * sigmaRang;

R.at(2, 2) = sigmaRang * sigmaRang;

// Scaled measurement noise

double sigmaPpos = 1.0;

double sigmaPang = sigmaPpos / a;

AlgebraicMatrix<double> RScaled(nmeas, nmeas);

for (int i = 0; i < nmeas; i++) {

for (int j = 0; j < nmeas; j++) { RScaled.at(i, j) = 0.0; }

}

RScaled.at(0, 0) = R.at(0, 0);

RScaled.at(1, 1) = R.at(1, 1);

RScaled.at(2, 2) = R.at(2, 2);

// **3. Noise Distributions**

std::default_random_engine generator;

std::normal_distribution<double> position_distribution(0.0, 87.88);

std::normal_distribution<double> velocity_distribution(0.0, 6.74e-4);

std::normal_distribution<double> distribution_r(0.0, sigmaRpos);

std::normal_distribution<double> distribution_ang(0.0, sigmaRang);

// **4. Initial Conditions**

AlgebraicVector<DA> x0(nvar), xc(nvar);

AlgebraicVector<double> x0_double(nvar);

x0[0] = -6045.0; x0[1] = -3490.0; x0[2] = 2500.0;

x0[3] = -3.457;  x0[4] = 6.618;   x0[5] = 2.533;

x0_double = cons(x0);

double sigmaPvel = 1.0;

for (int c = 0; c < 3; c++) xc[c] = x0[c] + sigmaPpos * DA(c + 1);

for (int c = 3; c < 6; c++) xc[c] = x0[c] + sigmaPvel * DA(c + 1);

// **5. Model Setup**

const int n_models = 3;

// Model Impulses

std::vector<Velocity> model_impulses(n_models);

model_impulses[0] = { 0.0, 0.0, 0.0 };  // No impulse

model_impulses[1] = { 0.0, 0.0, 0.30 }; // +0.3 km/s in z

model_impulses[2] = { 0.0, 0.0, -0.30 }; // -0.3 km/s in z

// True Impulse

Velocity true_impulse = { 0.0, 0.0, 0.3 }; // True scenario: +0.3 km/s in z

// **6. Monte Carlo Setup**

const int MC = 30;

const std::string state_vars[nvar] = { "x", "y", "z", "vx", "vy", "vz" };

// Storage Setup

std::vector<std::vector<std::vector<double>>> mc_states(MC, std::vector<std::vector<double>>(nObsPerIter, std::vector<double>(nvar, 0.0)));

std::vector<std::vector<std::vector<double>>> da_states(MC, std::vector<std::vector<double>>(nObsPerIter, std::vector<double>(nvar, 0.0)));

// Statistics storage

std::vector<std::vector<double>> da_means(nObsPerIter, std::vector<double>(nvar, 0.0));

std::vector<std::vector<double>> da_std_devs(nObsPerIter, std::vector<double>(nvar, 0.0));

// Metrics storage

std::vector<std::vector<double>> err_sum(nvar, std::vector<double>(nObsPerIter, 0.0));

std::vector<std::vector<double>> err_sq_sum(nvar, std::vector<double>(nObsPerIter, 0.0));

std::vector<std::vector<double>> cov_sum(nvar, std::vector<double>(nObsPerIter, 0.0));

// Weighted model storage

std::vector<double> weights(n_models, 1.0 / n_models); // Initial equal weights

std::vector<AlgebraicVector<double>> model_states(n_models, AlgebraicVector<double>(nvar));

std::vector<AlgebraicMatrix<double>> model_covs(n_models, AlgebraicMatrix<double>(nvar, nvar));

// Consolidated file streams

std::ofstream error_file("errors.csv");

std::ofstream weights_file("model_weights.csv");

std::ofstream pest_file("3sigma_Pest.csv"); // Renamed to reflect ±3sigma(Pest)

// Write headers

error_file << "time";

weights_file << "time";

pest_file << "time";

for (int i = 0; i < nvar; i++) {

error_file << "," << state_vars[i] << "_error";

pest_file << "," << state_vars[i] << "_plus_3sigma_Pest" << "," << state_vars[i] << "_minus_3sigma_Pest";

}

for (int m = 0; m < n_models; m++) {

weights_file << ",model" << m << "_weight";

}

error_file << "\n";

weights_file << "\n";

pest_file << "\n";

// Reset random seed for consistency

generator.seed(100);

// **7. Monte Carlo Loop**

for (int k = 0; k < MC; k++) {

std::cout << "Run Number: " << k + 1 << "/" << MC << std::endl;

AlgebraicVector<double> delta_x(nvar);

for (int c = 0; c < 3; c++) delta_x[c] = position_distribution(generator);

for (int c = 3; c < 6; c++) delta_x[c] = velocity_distribution(generator);

AlgebraicVector<double> current_mc = x0_double + delta_x;

AlgebraicVector<DA> xc_current = xc;

AlgebraicMatrix<double> P(nvar, nvar), PScaled(nvar, nvar);

AlgebraicMatrix<double> PZZ(nmeas, nmeas), PXZ(nvar, nmeas);

AlgebraicMatrix<double> INVPZZ(nmeas, nmeas), Kgain(nvar, nmeas);

AlgebraicMatrix<double> PUPD(nvar, nvar);

// Initial covariance matrix [P] set to 2% of state values

for (int i = 0; i < nvar; i++) {

for (int j = 0; j < nvar; j++) {

if (i == j) {

double state_value = std::abs(x0_double[i]);

P.at(i, j) = pow( .02*state_value, 2);//P.at(i, j) = pow(0.02 * state_value, 2);

}

else {

P.at(i, j) = 0.0;

}

}

}

PScaled = P;

// Initialize model states and covariances

for (int m = 0; m < n_models; m++) {

model_states[m] = x0_double + delta_x;

model_covs[m] = P;

}

AlgebraicVector<double> mn = x0_double;

AlgebraicVector<double> fused_state(nvar, 0.0);

AlgebraicMatrix<double> fused_cov(nvar, nvar, 0.0);

double t = 0.0;

for (int obs = 0; obs < nObsPerIter; obs++) {

double tk = t + dtspan;

// Propagate true state

DA::pushTO(filterorder);

current_mc = RK78(nvar, current_mc, t, tk, TBP<double>);

xc_current = RK78(nvar, xc_current, t, tk, TBP<DA>);

t = tk;

DA::popTO();

// Apply true impulse at T/2 (obs == 24)

if (obs == nObsPerIter/2) {

current_mc[3] += true_impulse.vx;

current_mc[4] += true_impulse.vy;

current_mc[5] += true_impulse.vz;

}

// Propagate each model with its impulse

for (int m = 0; m < n_models; m++) {

model_states[m] = RK78(nvar, model_states[m], t - dtspan, t, TBP<double>);

if (obs == nObsPerIter/2) {

model_states[m][3] += model_impulses[m].vx;

model_states[m][4] += model_impulses[m].vy;

model_states[m][5] += model_impulses[m].vz;

}

}

// Measurements

const auto& st = current_mc;

AlgebraicVector<double> ztrue(nmeas), zpred(nmeas), zdiff(nmeas), mnupdate(nvar);

AlgebraicVector<DA> zf(nmeas);

// Generate true measurements with noise

ztrue[0] = sqrt(st[0] * st[0] + st[1] * st[1] + st[2] * st[2]) + distribution_r(generator);

ztrue[1] = atan2(st[1], st[0]) + distribution_ang(generator);

ztrue[2] = asin(st[2] / ztrue[0]) + distribution_ang(generator);

DA::pushTO(filterorder);

DA::pushTO(2 * filterorder);

// Moments computation

DA PscaledPol = 0;

for (int i = 0; i < nvar; i++) {

for (int j = i; j < nvar; j++) {

PscaledPol += PScaled.at(i, j) * DA(i + 1) * DA(j + 1);

}

}

DA moments = computeMoments(PscaledPol);

// Predict measurements for DA (single model for now)

AlgebraicVector<DA> xf = xc_current;

zf[0] = sqrt(xf[0] * xf[0] + xf[1] * xf[1] + xf[2] * xf[2]);

zf[1] = atan2(xf[1], xf[0]);

zf[2] = asin(xf[2] / zf[0]);

zpred[0] = zf[0].evalMonomials(moments);

zpred[1] = zf[1].evalMonomials(moments);

zpred[2] = zf[2].evalMonomials(moments);

// Weight update for models

std::vector<double> likelihoods(n_models, 0.0);

double weight_sum = 0.0;

for (int m = 0; m < n_models; m++) {

AlgebraicVector<double> zpred_model(nmeas);

const auto& sm = model_states[m];

zpred_model[0] = sqrt(sm[0] * sm[0] + sm[1] * sm[1] + sm[2] * sm[2]);

zpred_model[1] = atan2(sm[1], sm[0]);

zpred_model[2] = asin(sm[2] / zpred_model[0]);

AlgebraicVector<double> innovation = ztrue - zpred_model;

double exponent = 0.0;

for (int i = 0; i < nmeas; i++) {

exponent += -0.5 * innovation[i] * innovation[i] / R.at(i, i);

}

likelihoods[m] = exp(exponent);

weights[m] *= likelihoods[m];

weight_sum += weights[m];

}

// Normalize weights

for (int m = 0; m < n_models; m++) {

weights[m] /= weight_sum;

}

// Compute fused state and covariance

fused_state = AlgebraicVector<double>(nvar, 0.0);

fused_cov = AlgebraicMatrix<double>(nvar, nvar, 0.0);

for (int m = 0; m < n_models; m++) {

for (int i = 0; i < nvar; i++) {

fused_state[i] += weights[m] * model_states[m][i];

}

AlgebraicVector<double> diff = model_states[m] - fused_state;

for (int i = 0; i < nvar; i++) {

for (int j = 0; j < nvar; j++) {

fused_cov.at(i, j) += weights[m] * (model_covs[m].at(i, j) + diff[i] * diff[j]);

}

}

}

// Original DA update continues

DA::pushTO(2 * filterorder);

for (int i = 0; i < nvar; i++) {

for (int j = i; j < nvar; j++) {

PscaledPol += PScaled.at(i, j) * DA(i + 1) * DA(j + 1);

}

}

for (int i = 0; i < nvar; i++) {

mn[i] = xf[i].evalMonomials(moments);

}

for (int i = 0; i < 3; i++) {

xf[i] = (xf[i] - mn[i]);

xf[i + 3] = (xf[i + 3] - mn[i + 3]);

}

zf[0] = (zf[0] - zpred[0]);

zf[1] = (zf[1] - zpred[1]);

zf[2] = (zf[2] - zpred[2]);

AlgebraicMatrix<DA> xx(nvar, nvar), zz(nmeas, nmeas), xz(nvar, nmeas);

for (int i = 0; i < nvar; i++) {

for (int j = i; j < nvar; j++) {

xx.at(i, j) = xf[i] * xf[j];

PUPD.at(i, j) = xx.at(i, j).evalMonomials(moments);

PUPD.at(j, i) = PUPD.at(i, j);

}

}

for (int i = 0; i < nmeas; i++) {

for (int j = i; j < nmeas; j++) {

zz.at(i, j) = zf[i] * zf[j];

PZZ.at(i, j) = zz.at(i, j).evalMonomials(moments);

PZZ.at(j, i) = PZZ.at(j, i);

}

}

PZZ = PZZ + RScaled;

for (int i = 0; i < nvar; i++) {

for (int j = 0; j < nmeas; j++) {

xz.at(i, j) = xf[i] * zf[j];

PXZ.at(i, j) = xz.at(i, j).evalMonomials(moments);

}

}

PScaled = PUPD;

double det_PZZ = PZZ.det();

if (std::abs(det_PZZ) < 1e-8) {

std::cout << "Warning: PZZ nearly singular, det = " << det_PZZ << std::endl;

for (int i = 0; i < nmeas; i++) PZZ.at(i, i) += 1e-4;

}

INVPZZ = PZZ.inv();

Kgain = PXZ * INVPZZ;

zdiff = ztrue - zpred;

for (int i = 0; i < nvar; i++) {

mnupdate[i] = 0;

for (int j = 0; j < nmeas; j++) {

mnupdate[i] += Kgain.at(i, j) * zdiff[j];

}

mn[i] = mn[i] + mnupdate[i];

}

AlgebraicMatrix<double> GG = Kgain * PZZ * Kgain.transpose();

PScaled = PScaled - GG;

for (int i = 0; i < nvar; i++) {

for (int j = 0; j < nvar; j++) {

if (i != j) PScaled.at(j, i) = PScaled.at(i, j);

}

}

if (k == 0) {

for (int i = 0; i < nvar; i++) {

da_means[obs][i] = fused_state[i];

da_std_devs[obs][i] = sqrt(fused_cov.at(i, i));

}

}

DA::popTO();

// Compute and store metrics using fused state

error_file << t;

pest_file << t;

weights_file << t;

for (int i = 0; i < nvar; i++) {

double error =  fused_state[i]- current_mc[i];

err_sum[i][obs] += error;

err_sq_sum[i][obs] += error * error;

error_file << "," << error;

double pest_variance = fused_cov.at(i, i); // Pest is the diagonal of fused_cov

cov_sum[i][obs] += sqrt(pest_variance);

pest_file << "," << 3 * sqrt(pest_variance) << "," << -3 * sqrt(pest_variance); // ±3sigma(Pest)

}

for (int m = 0; m < n_models; m++) {

weights_file << "," << weights[m];

}

error_file << "\n";

pest_file << "\n";

weights_file << "\n";

} // End of observation loop

} // End of Monte Carlo loop

// **8. Write Metrics to CSV Files**

for (int obs = 0; obs < nObsPerIter; obs++) {

for (int i = 0; i < nvar; i++) {

double mean_err = err_sum[i][obs] / MC;

double sum_err = err_sum[i][obs];

double sum_sq_err = err_sq_sum[i][obs];

double variance = (sum_sq_err - (sum_err * sum_err / MC)) / (MC - 1);

double std_dev = sqrt(variance);

double cov_3sigma = 3 * (cov_sum[i][obs] / MC);

// Note: These metrics are no longer written here as they're in pest_file and error_file

}

}

// **9. Clean Up**

error_file.close();

weights_file.close();

pest_file.close();

std::cout << "Simulation complete. Data written to CSV files.\n";

return 0;

}