int main() {

struct Velocity {

double vx, vy, vz;

};

// **1. Initialize DACE**

DA::setEps(0.0);

const int filterorder = 2;

const int nvar = 6;

DA::init(2 * filterorder, nvar);

// **2. Orbital Parameters**

const double mu = 398600.0;

const double pi = 3.14159265358979323846;

const double a = 8788.0;

const double T = 2.0 * pi * sqrt(pow(a, 3) / mu);

const int nObsPerIter = 10;

const double dtspan = T / nObsPerIter;

const int nmeas = 3;

// Measurement noise covariance [R]

const double sigmaRpos = 1e-1;

const double sigmaRang = (100 / 3600.0) * pi / 180.0;

AlgebraicMatrix<double> R(nmeas, nmeas);

for (int i = 0; i < nmeas; i++) for (int j = 0; j < nmeas; j++) R.at(i, j) = 0.0;

R.at(0, 0) = sigmaRpos * sigmaRpos;

R.at(1, 1) = sigmaRang * sigmaRang;

R.at(2, 2) = sigmaRang * sigmaRang;

// **3. Noise Distributions**

default_random_engine generator;

generator.seed(100);

normal_distribution<double> distribution_r(0.0, sigmaRpos);

normal_distribution<double> distribution_ang(0.0, sigmaRang);

normal_distribution<double> position_distribution(0.0, 87.88);

normal_distribution<double> velocity_distribution(0.0, 6.74e-4);

// **4. Initial Conditions**

AlgebraicVector<DA> x0(nvar), xc(nvar);

AlgebraicVector<double> x0_double(nvar);

x0[0] = -6045.0; x0[1] = -3490.0; x0[2] = 2500.0;

x0[3] = -3.457;  x0[4] = 6.618;   x0[5] = 2.533;

x0_double = cons(x0);

double sigmaPpos = 1.0;

double sigmaPvel = 1.0;

for (int c = 0; c < 3; c++) xc[c] = x0[c] + sigmaPpos * DA(c + 1);

for (int c = 3; c < 6; c++) xc[c] = x0[c] + sigmaPvel * DA(c + 1);

// **5. Velocity Perturbations**

const double V = 0.3;

vector<double> azimuth = { 0, 90, 180, 270 };

vector<double> elevation = { -90, -45, 0, 45, 90 };

vector<Velocity> delta_v_combinations;

for (double theta_deg : azimuth) {

for (double phi_deg : elevation) {

double theta = theta_deg * pi / 180.0;

double phi = phi_deg * pi / 180.0;

Velocity v;

v.vx = V * cos(theta) * cos(phi);

v.vy = V * sin(theta) * cos(phi);

v.vz = V * sin(phi);

delta_v_combinations.push_back(v);

}

}

// **6. Storage Setup**

const int num_perturbations = delta_v_combinations.size();

const int MC = 50;

vector<vector<vector<vector<double>>>> perturbed_mc_states(num_perturbations,

vector<vector<vector<double>>>(MC, vector<vector<double>>(nObsPerIter, vector<double>(nvar, 0.0))));

vector<vector<vector<vector<double>>>> perturbed_da_states(num_perturbations,

vector<vector<vector<double>>>(MC, vector<vector<double>>(nObsPerIter, vector<double>(nvar, 0.0))));

vector<vector<vector<double>>> da_std_devs(num_perturbations,

vector<vector<double>>(nObsPerIter, vector<double>(nvar, 0.0)));

vector<vector<vector<double>>> mc_means(num_perturbations,

vector<vector<double>>(nObsPerIter, vector<double>(nvar, 0.0)));

vector<vector<vector<double>>> mc_std_devs(num_perturbations,

vector<vector<double>>(nObsPerIter, vector<double>(nvar, 0.0)));

// **7. Main Loops**

for (int perturbation_idx = 0; perturbation_idx < num_perturbations; perturbation_idx++) {

Velocity perturbation = delta_v_combinations[perturbation_idx];

int az_idx = perturbation_idx / elevation.size();

int el_idx = perturbation_idx % elevation.size();

cout << "Processing perturbation " << perturbation_idx + 1 << "/" << num_perturbations

<< " (Azimuth: " << azimuth[az_idx] << "°, Elevation: " << elevation[el_idx] << "°)" << endl;

AlgebraicVector<double> perturbed_x0_double = x0_double;

perturbed_x0_double[3] += perturbation.vx;

perturbed_x0_double[4] += perturbation.vy;

perturbed_x0_double[5] += perturbation.vz;

AlgebraicVector<DA> perturbed_xc = xc;

perturbed_xc[3] += perturbation.vx;

perturbed_xc[4] += perturbation.vy;

perturbed_xc[5] += perturbation.vz;

// Monte Carlo Loop

for (int k = 0; k < MC; k++) {

cout << "  Run Number: " << k + 1 << "/" << MC << endl;

double t = 0.0;

AlgebraicVector<double> delta_x(nvar);

for (int c = 0; c < 3; c++) delta_x[c] = position_distribution(generator);

for (int c = 3; c < 6; c++) delta_x[c] = velocity_distribution(generator);

AlgebraicVector<double> current_mc = perturbed_x0_double + delta_x;

AlgebraicVector<DA> xc_current = perturbed_xc;

AlgebraicMatrix<double> P(nvar, nvar), PScaled(nvar, nvar);

for (int i = 0; i < nvar; i++) {

for (int j = 0; j < nvar; j++) {

if (i == j) {

double state_value = abs(perturbed_x0_double[i]);

P.at(i, j) = pow(0.02 * state_value, 2);

}

else {

P.at(i, j) = 0.0;

}

}

}

PScaled = P;

AlgebraicVector<double> mn = perturbed_x0_double;

// Observation Loop - PREDICTION ONLY

for (int obs = 0; obs < nObsPerIter; obs++) {

double tk = t + dtspan;

// Reset DA state with current mean

for (int i = 0; i < nvar; i++) {

xc_current[i] = mn[i] + DA(i + 1);

}

// PREDICTION STEP

DA::pushTO(filterorder);

current_mc = RK78(nvar, current_mc, t, tk, TBP<double>);

xc_current = RK78(nvar, xc_current, t, tk, TBP<DA>);

t = tk;

DA::popTO();

// Store states

for (int i = 0; i < nvar; i++) {

perturbed_mc_states[perturbation_idx][k][obs][i] = current_mc[i];

}

// DA Covariance Prediction

DA::pushTO(2 * filterorder);

DA PscaledPol = 0;

for (int i = 0; i < nvar; i++) {

for (int k = i; k < nvar; k++) {

PscaledPol += PScaled.at(i, k) * DA(i + 1) * DA(k + 1);

}

}

DA moments = computeMoments(PscaledPol); // Placeholder function

AlgebraicVector<double> da_mean(nvar);

for (int i = 0; i < nvar; i++) {

da_mean[i] = xc_current[i].evalMonomials(moments);

perturbed_da_states[perturbation_idx][k][obs][i] = da_mean[i];

}

AlgebraicVector<DA> xf = xc_current;

for (int i = 0; i < nvar; i++) {

xf[i] = xf[i] - da_mean[i];

}

AlgebraicMatrix<DA> xx(nvar, nvar);

AlgebraicMatrix<double> PUPD(nvar, nvar);

for (int i = 0; i < nvar; i++) {

for (int k = i; k < nvar; k++) {

xx.at(i, k) = xf[i] * xf[k];

PUPD.at(i, k) = xx.at(i, k).evalMonomials(moments);

PUPD.at(k, i) = PUPD.at(i, k);

}

}

// Store the predicted covariance

for (int i = 0; i < nvar; i++) {

da_std_devs[perturbation_idx][obs][i] = sqrt(PUPD.at(i, i));

}

// Update for next step (using predicted values)

mn = da_mean;

PScaled = PUPD;

DA::popTO();

}

}

// Monte Carlo Statistics

for (int obs = 0; obs < nObsPerIter; obs++) {

for (int i = 0; i < nvar; i++) {

double sum = 0.0;

for (int k = 0; k < MC; k++) {

sum += perturbed_mc_states[perturbation_idx][k][obs][i];

}

mc_means[perturbation_idx][obs][i] = sum / MC;

double sum_sq_diff = 0.0;

for (int k = 0; k < MC; k++) {

double diff = perturbed_mc_states[perturbation_idx][k][obs][i] - mc_means[perturbation_idx][obs][i];

sum_sq_diff += diff * diff;

}

mc_std_devs[perturbation_idx][obs][i] = sqrt(sum_sq_diff / MC);

}

}

}

// **8. Write Results**

const string state_vars[6] = { "x", "y", "z", "vx", "vy", "vz" };

// Monte Carlo and DA States (unchanged from original)

for (int i = 0; i < nvar; i++) {

string filename = state_vars[i] + "_mc_all_perturbations.csv";

ofstream file(filename);

file << "Run";

for (int p = 0; p < num_perturbations; p++) {

int az_idx = p / elevation.size();

int el_idx = p % elevation.size();

string az_str = to_string(static_cast<int>(azimuth[az_idx]));

string el_str = to_string(static_cast<int>(elevation[el_idx]));

for (int obs = 0; obs < nObsPerIter; obs++) {

file << ",Az" << az_str << "El" << el_str << "_Obs" << obs;

}

}

file << "\n";

file << fixed << setprecision(16);

for (int k = 0; k < MC; k++) {

file << k + 1;

for (int p = 0; p < num_perturbations; p++) {

for (int obs = 0; obs < nObsPerIter; obs++) {

file << "," << perturbed_mc_states[p][k][obs][i];

}

}

file << "\n";

}

file.close();

filename = state_vars[i] + "_da_all_perturbations.csv";

ofstream file_da(filename);

file_da << "Run";

for (int p = 0; p < num_perturbations; p++) {

int az_idx = p / elevation.size();

int el_idx = p % elevation.size();

string az_str = to_string(static_cast<int>(azimuth[az_idx]));

string el_str = to_string(static_cast<int>(elevation[el_idx]));

for (int obs = 0; obs < nObsPerIter; obs++) {

file_da << ",Az" << az_str << "El" << el_str << "_Obs" << obs;

}

}

file_da << "\n";

file_da << fixed << setprecision(16);

for (int k = 0; k < MC; k++) {

file_da << k + 1;

for (int p = 0; p < num_perturbations; p++) {

for (int obs = 0; obs < nObsPerIter; obs++) {

file_da << "," << perturbed_da_states[p][k][obs][i];

}

}

file_da << "\n";

}

file_da.close();

}

// Covariance Comparison

for (int i = 0; i < nvar; i++) {

string filename = state_vars[i] + "_covariance_comparison.csv";

ofstream file(filename);

file << "Perturbation,Observation,MC_Mean,MC_StdDev,MC_Upper3Sigma,MC_Lower3Sigma,DA_StdDev,DA_Upper3Sigma,DA_Lower3Sigma\n";

file << fixed << setprecision(16);

for (int p = 0; p < num_perturbations; p++) {

int az_idx = p / elevation.size();

int el_idx = p % elevation.size();

string label = "Az" + to_string(static_cast<int>(azimuth[az_idx])) + "El" + to_string(static_cast<int>(elevation[el_idx]));

for (int obs = 0; obs < nObsPerIter; obs++) {

double mc_mean = mc_means[p][obs][i];

double mc_std = mc_std_devs[p][obs][i];

double da_std = da_std_devs[p][obs][i];

file << label << "," << obs << "," << mc_mean << "," << mc_std << ","

<< mc_mean + 3 * mc_std << "," << mc_mean - 3 * mc_std << ","

<< da_std << "," << mc_mean + 3 * da_std << "," << mc_mean - 3 * da_std << "\n";

}

}

file.close();

}

cout << "Simulation complete. Data written to CSV files.\n";

return 0;

}