#define WITH_ALGEBRAICMATRIX

#include "dace/dace.h"

#include <cmath>

#include <iostream>

#include <fstream>

#include <iomanip>

#include <random>

#include <chrono>

#include <numeric>

// for random numbers

using namespace std;

using namespace DACE;

// Random number generator for normal distribution

std::default_random_engine generator;

std::normal_distribution<double> position_distribution(0.0, 87.88);  // mean = 0, stddev = 87.88 km

std::normal_distribution<double> velocity_distribution(0.0, 6.74e-4);  // mean = 0, stddev = 6.74e-4 km/s

template<typename T> AlgebraicVector<T> TBP(AlgebraicVector<T> x, double t)

{

AlgebraicVector<T> pos(3), res(6);

pos[0] = x[0]; pos[1] = x[1]; pos[2] = x[2];

T r = sqrt(sqr(pos[0]) + sqr(pos[1]) + sqr(pos[2]));

const double mu = 398600.0;

res[0] = x[3];

res[1] = x[4];

res[2] = x[5];

res[3] = -mu * pos[0] / (r * r * r);

res[4] = -mu * pos[1] / (r * r * r);

res[5] = -mu * pos[2] / (r * r * r);

return res;

}

#if DACE_MAJOR_VERSION==2

typedef unsigned int exponent_type;

#else

typedef int exponent_type;		// work around bug in old DACE 1 interface

#endif

double min(double a, double b) {

return (a < b) ? a : b;

}

double max(double a, double b) {

return (a > b) ? a : b;

}

double normtmp(int N, vector<double> X) {

double res = 0.0;

for (int I = 0; I < N; I++) {

if (X[I] < 0) { X[I] = -X[I]; }

res = max(res, X[I]);

}

return res;

}

template<typename T> AlgebraicVector<T> RK78(int N, AlgebraicVector<T> Y0, double X0, double X1, AlgebraicVector<T>(*f)(AlgebraicVector<T>, double))

{

double ERREST;

double H0 = .001; double HS = 0.1; double H1 = 100.0;

double EPS = 1.e-12; double BS = 20 * EPS;

int I, J, K;

// AlgebraicMatrix<T> Z(N,16);

vector< vector<T> > Z;

for (I = 0; I < N; I++)

{

vector<T> tmp(16);

Z.push_back(tmp);

}

AlgebraicVector<T> Y1(N);

AlgebraicVector<T> out(N);

for (int k = 0; k < N; k++) { out[k] = 0.0; }

vector<double> Y1cons(N);

double VIHMAX = 0.0, X, H;

double RFNORM, HH0, HH1;

double HSQR = 1.0 / 9.0;

double A[13], C[13], D[13];

double B[13][12];

A[0] = 0.0; A[1] = 1.0 / 18.0; A[2] = 1.0 / 12.0; A[3] = 1.0 / 8.0; A[4] = 5.0 / 16.0; A[5] = 3.0 / 8.0;

A[6] = 59.0 / 400.0; A[7] = 93.0 / 200.0; A[8] = 5490023248.0 / 9719169821.0; A[9] = 13.0 / 20.0; A[10] = 1201146811.0 / 1299019798.0; A[11] = 1.0;

A[12] = 1.0;

B[0][0] = 0.0; B[0][1] = 0.0; B[0][2] = 0.0; B[0][3] = 0.0; B[0][4] = 0.0;

B[0][5] = 0.0; B[0][6] = 0.0; B[0][7] = 0.0; B[0][8] = 0.0; B[0][9] = 0.0;

B[0][10] = 0.0; B[0][11] = 0.0;

B[1][0] = 1.0 / 18.0; B[1][1] = 0.0; B[1][2] = 0.0; B[1][3] = 0.0; B[1][4] = 0.0;

B[1][5] = 0.0; B[1][6] = 0.0; B[1][7] = 0.0; B[1][8] = 0.0; B[1][9] = 0.0;

B[1][10] = 0.0; B[1][11] = 0.0;

B[2][0] = 1.0 / 48.0; B[2][1] = 1.0 / 16.0; B[2][2] = 0.0; B[2][3] = 0.0; B[2][4] = 0.0;

B[2][5] = 0.0; B[2][6] = 0.0; B[2][7] = 0.0; B[2][8] = 0.0; B[2][9] = 0.0;

B[2][10] = 0.0; B[2][11] = 0.0;

B[3][0] = 1.0 / 32.0; B[3][1] = 0.0; B[3][2] = 3.0 / 32.0; B[3][3] = 0.0; B[3][4] = 0.0;

B[3][5] = 0.0; B[3][6] = 0.0; B[3][7] = 0.0; B[3][8] = 0.0; B[3][9] = 0.0;

B[3][10] = 0.0; B[3][11] = 0.0;

B[4][0] = 5.0 / 16.0; B[4][1] = 0.0; B[4][2] = -75.0 / 64.0; B[4][3] = 75.0 / 64.0; B[4][4] = 0.0;

B[4][5] = 0.0; B[4][6] = 0.0; B[4][7] = 0.0; B[4][8] = 0.0; B[4][9] = 0.0;

B[4][10] = 0.0; B[4][11] = 0.0;

B[5][0] = 3.0 / 80.0; B[5][1] = 0.0; B[5][2] = 0.0; B[5][3] = 3.0 / 16.0; B[5][4] = 3.0 / 20.0;

B[5][5] = 0.0; B[5][6] = 0.0; B[5][7] = 0.0; B[5][8] = 0.0; B[5][9] = 0.0;

B[5][10] = 0.0; B[5][11] = 0.0;

B[6][0] = 29443841.0 / 614563906.0; B[6][1] = 0.0; B[6][2] = 0.0; B[6][3] = 77736538.0 / 692538347.0; B[6][4] = -28693883.0 / 1125000000.0;

B[6][5] = 23124283.0 / 1800000000.0; B[6][6] = 0.0; B[6][7] = 0.0; B[6][8] = 0.0; B[6][9] = 0.0;

B[6][10] = 0.0; B[6][11] = 0.0;

B[7][0] = 16016141.0 / 946692911.0; B[7][1] = 0.0; B[7][2] = 0.0; B[7][3] = 61564180.0 / 158732637.0; B[7][4] = 22789713.0 / 633445777.0;

B[7][5] = 545815736.0 / 2771057229.0; B[7][6] = -180193667.0 / 1043307555.0; B[7][7] = 0.0; B[7][8] = 0.0; B[7][9] = 0.0;

B[7][10] = 0.0; B[7][11] = 0.0;

B[8][0] = 39632708.0 / 573591083.0; B[8][1] = 0.0; B[8][2] = 0.0; B[8][3] = -433636366.0 / 683701615.0; B[8][4] = -421739975.0 / 2616292301.0;

B[8][5] = 100302831.0 / 723423059.0; B[8][6] = 790204164.0 / 839813087.0; B[8][7] = 800635310.0 / 3783071287.0; B[8][8] = 0.0; B[8][9] = 0.0;

B[8][10] = 0.0; B[8][11] = 0.0;

B[9][0] = 246121993.0 / 1340847787.0; B[9][1] = 0.0; B[9][2] = 0.0; B[9][3] = -37695042795.0 / 15268766246.0; B[9][4] = -309121744.0 / 1061227803.0;

B[9][5] = -12992083.0 / 490766935.0; B[9][6] = 6005943493.0 / 2108947869.0; B[9][7] = 393006217.0 / 1396673457.0; B[9][8] = 123872331.0 / 1001029789.0; B[9][9] = 0.0;

B[9][10] = 0.0; B[9][11] = 0.0;

B[10][0] = -1028468189.0 / 846180014.0; B[10][1] = 0.0; B[10][2] = 0.0; B[10][3] = 8478235783.0 / 508512852.0; B[10][4] = 1311729495.0 / 1432422823.0;

B[10][5] = -10304129995.0 / 1701304382.0; B[10][6] = -48777925059.0 / 3047939560.0; B[10][7] = 15336726248.0 / 1032824649.0; B[10][8] = -45442868181.0 / 3398467696.0; B[10][9] = 3065993473.0 / 597172653.0;

B[10][10] = 0.0; B[10][11] = 0.0;

B[11][0] = 185892177.0 / 718116043.0; B[11][1] = 0.0; B[11][2] = 0.0; B[11][3] = -3185094517.0 / 667107341.0; B[11][4] = -477755414.0 / 1098053517.0;

B[11][5] = -703635378.0 / 230739211.0; B[11][6] = 5731566787.0 / 1027545527.0; B[11][7] = 5232866602.0 / 850066563.0; B[11][8] = -4093664535.0 / 808688257.0; B[11][9] = 3962137247.0 / 1805957418.0;

B[11][10] = 65686358.0 / 487910083.0; B[11][11] = 0.0;

B[12][0] = 403863854.0 / 491063109.0; B[12][1] = 0.0; B[12][2] = 0.0; B[12][3] = -5068492393.0 / 434740067.0; B[12][4] = -411421997.0 / 543043805.0;

B[12][5] = 652783627.0 / 914296604.0; B[12][6] = 11173962825.0 / 925320556.0; B[12][7] = -13158990841.0 / 6184727034.0; B[12][8] = 3936647629.0 / 1978049680.0; B[12][9] = -160528059.0 / 685178525.0;

B[12][10] = 248638103.0 / 1413531060.0; B[12][11] = 0.0;

C[0] = 14005451.0 / 335480064.0; C[1] = 0.0; C[2] = 0.0; C[3] = 0.0; C[4] = 0.0; C[5] = -59238493.0 / 1068277825.0;

C[6] = 181606767.0 / 758867731.0; C[7] = 561292985.0 / 797845732.0; C[8] = -1041891430.0 / 1371343529.0; C[9] = 760417239.0 / 1151165299.0; C[10] = 118820643.0 / 751138087.0; C[11] = -528747749.0 / 2220607170.0;

C[12] = 1.0 / 4.0;

D[0] = 13451932.0 / 455176623.0; D[1] = 0.0; D[2] = 0.0; D[3] = 0.0; D[4] = 0.0; D[5] = -808719846.0 / 976000145.0;

D[6] = 1757004468.0 / 5645159321.0; D[7] = 656045339.0 / 265891186.0; D[8] = -3867574721.0 / 1518517206.0; D[9] = 465885868.0 / 322736535.0; D[10] = 53011238.0 / 667516719.0; D[11] = 2.0 / 45.0;

D[12] = 0.0;

for (I = 0; I < N; I++)

{

Z[I][0] = Y0[I];

Z[I][1] = 0.0;

}

H = abs(HS); HH0 = abs(H0); HH1 = abs(H1);

if (X1 - X0 < 0)

{

H = -H;

HH0 = -HH0;

HH1 = -HH1;

};

X = X0; RFNORM = 0.0; ERREST = 0.0;

while (X != X1) {

// compute new stepsize

if (RFNORM != 0) { H = H * min(4.0, exp(HSQR * log(EPS / RFNORM))); }

if (abs(H) > abs(HH1)) { H = HH1; }

else if (abs(H) < abs(HH0) * 0.99) {

H = HH0;

cout << "--- WARNING, MINIMUM STEPSIZE REACHED IN RK" << endl;

return out;

}

if ((X + H - X1) * H > 0) { H = X1 - X; }

for (J = 0; J < 13; J++) {

for (I = 0; I < N; I++) {

Y0[I] = 0.0; // EVALUATE RHS AT 13 POINTS

for (K = 0; K < J; K++) { Y0[I] = Y0[I] + Z[I][K + 3] * B[J][K]; }

Y0[I] = H * Y0[I] + Z[I][0];

}

Y1 = f(Y0, X + H * A[J]);

for (I = 0; I < N; I++) { Z[I][J + 3] = Y1[I]; }

}

for (I = 0; I < N; I++) {

Z[I][1] = 0.0; Z[I][2] = 0.0; // EXECUTE 7TH,8TH ORDER STEPS

for (J = 0; J < 13; J++) {

Z[I][1] = Z[I][1] + Z[I][J + 3] * D[J];

Z[I][2] = Z[I][2] + Z[I][J + 3] * C[J];

}

Y1[I] = (Z[I][2] - Z[I][1]) * H;

Z[I][2] = Z[I][2] * H + Z[I][0];

}

for (I = 0; I < N; I++) { Y1cons[I] = cons(Y1[I]); }

RFNORM = normtmp(N, Y1cons); // ESTIMATE ERROR AND DECIDE ABOUT BACKSTEP

if ((RFNORM > BS) && (abs(H / H0) > 1.2)) {

H = H / 3.0;

RFNORM = 0;

}

else {

for (I = 0; I < N; I++) { Z[I][0] = Z[I][2]; }

X = X + H;

VIHMAX = max(VIHMAX, H);

ERREST = ERREST + RFNORM;

}

}

for (I = 0; I < N; I++) { Y1[I] = Z[I][0]; }

return Y1;

}

double computeMonomialEV(DA& cov, DA& moments, std::vector<unsigned int> p)

{

unsigned int i0 = 0;

const unsigned int nomax = DA::getMaxVariables();

unsigned int order = 0;

// find any non-zero exponent i0 and while doing so compute the order of p

for (unsigned int i = 0; i < nomax; i++)

{

if (p[i] > 0) i0 = i;

order += p[i];

}

// if order is zero the expectation value is 1

if (order == 0)

return 1.0;

// if order is odd, the expectation value is 0

else if (order & 1)

return 0.0;

// find first non-zero exponent i0 (assumed to exist, also order assumed to be even!)

// !!!!!!!!!! check !!!!!!!!!!!!!

for (i0 = 0; i0 < nomax; i0++)

if (p[i0] > 0)

break;

// now we know that order is even and i0 is a non-zero exponent

std::vector<unsigned int> q(nomax, 0);

// remove the non-zero exponent from p and move it to q

q[i0] = 1;

p[i0]--;

double res = 0.0;

// run through all variables.

// This loop will enumerate all possible unordered pairs of 2 variables and O(p)-2 variables, and hence compute the expectation value according to a recursive Isserlis formula.

for (unsigned int i = 0; i < nomax; i++)

{

// if the exponent in p is zero skip this variable

if (p[i] == 0) continue;

// move one exponent of variable i from p to q

q[i]++;

p[i]--;

// compute the contribution of the pair E[q]*E[p] where q is a covariance (order 2) and p is a monomial of order O(p)-2

// also account for the multiplicity of values due to the power of the i-th variable

res += cov.getCoefficient(q) * moments.getCoefficient(p) * (p[i] + 1);

// move exponent back

q[i]--;

p[i]++;

}

p[i0]++;	// return p into its original state

return res;

}

DA computeMoments(DA& cov)

{

const unsigned int nomax = DA::getMaxOrder();

DA pairs;

for (unsigned int i = 0; i < DA::getMaxVariables(); i++)

pairs += DA(i + 1);	// now contains every possible monomial of exact order 1

pairs *= pairs;			// now contains every possible monomial of exact order 2

DA res = 1.0 + cov,		// the result will contain the expectation value of each monomial, starting with 1 for constant part and provided covariances

o2n = pairs * pairs,	// now contains every possible monomial of order 4

prev = cov;			// now contains all expectation values of monomials of order 2 (i.e. the covariance)

for (unsigned int i = 4; i <= nomax; i += 2, o2n *= pairs)

{

DA temp;	// here we sum up all the expectation values of monomials of exact order i

// enumerate all monomials in o2n, which is all possible monomials of exact order i

const unsigned int jmax = o2n.size();

for (unsigned int j = 1; j <= jmax; j++)		// monomial indices are one based!

{

const Monomial m = o2n.getMonomial(j);		// get the j-th monomial

temp.setCoefficient(m.m_jj, computeMonomialEV(cov, prev, m.m_jj));	// compute its expectation value using the covariance and the i-1 order expectation values and store it in the corresponding monomial in temp

}

res += temp;	// add all expectation values of order i to the result

std::swap(prev, temp);	// store all expectation values of order i in prev for the next loop

}

return res;

}

int main()

{

// 1. Initialize DACE

int filterorder = 1;

DA::init(2 * filterorder, 6);

// 2. Orbital parameters

const double mu = 398600.0;

const double pi = 3.141592653589793;

const double a = 8788.0; // [km]

const double T = 2.0 * pi * sqrt(pow(a, 3) / mu);

const int nObs = 200;    // Observations per iteration

const int nIters = 500;  // Monte Carlo draws

const double dtspan = T / nObs;

// 3. Nominal initial state

AlgebraicVector<double> x0_double(6);

x0_double[0] = -6045.0;

x0_double[1] = -3490.0;

x0_double[2] = 2500.0;

x0_double[3] = -3.457;

x0_double[4] = 6.618;

x0_double[5] = 2.533;

// 4. Create output files ONCE before the main loop

ofstream outfiles[6];

const string components[6] = { "x", "y", "z", "vx", "vy", "vz" };

for (int i = 0; i < 6; i++) {

outfiles[i].open(components[i] + ".csv");

if (!outfiles[i]) {

cerr << "Error opening " << components[i] << ".csv" << endl;

return 1;

}

}

// 5. Main Monte Carlo loop

for (int iIter = 0; iIter < nIters; ++iIter) {

// Generate perturbed initial state

AlgebraicVector<double> delta_x(6);

for (int i = 0; i < 3; i++) delta_x[i] = position_distribution(generator);

for (int i = 3; i < 6; i++) delta_x[i] = velocity_distribution(generator);

AlgebraicVector<double> x_perturbed = x0_double + delta_x;

// Propagate through full orbit

AlgebraicVector<double> prev_state = x_perturbed;

for (int obs = 0; obs < nObs; obs++) {

const double current_time = obs * dtspan;

const AlgebraicVector<double> state = RK78(6, prev_state, 0.0, dtspan, TBP<double>);

prev_state = state;

// Write to files

for (int comp = 0; comp < 6; comp++) {

outfiles[comp] << state[comp];

if (obs < nObs - 1) outfiles[comp] << ",";

}

}

// Add newline after each iteration

for (int comp = 0; comp < 6; comp++) {

outfiles[comp] << "\n";

}

// Progress reporting

if ((iIter + 1) % 50 == 0) {

cout << "Completed " << (iIter + 1) << "/" << nIters << " iterations" << endl;

}

}

// 6. Cleanup

for (int i = 0; i < 6; i++) {

outfiles[i].close();

}

cout << "Propagation complete. Created 6 files with "

<< nIters << "x" << nObs << " matrices." << endl;

return 0;

}