% Enhanced MATLAB Script to Visualize ALL Models with Different Shading Techniques

% This script shows all models' trajectories, ellipsoids, and uncertainty in unified views

% Uses advanced visualization techniques for clarity when showing many models

%------------------------for half ------------------

clear all; close all; clc;

% Define the CSV filenames for model data

model_data_files = {'model_data_x.csv', 'model_data_y.csv', 'model_data_z.csv'};

labels = {'X Position', 'Y Position', 'Z Position'};

% Orbital parameters (matching C++ code)

mu = 398600.0;

a = 8788.0;

T = 2.0 * pi * sqrt(a^3 / mu);

nObsPerIter = 24;

dtspan = T / nObsPerIter;

half_revolution_start = floor(nObsPerIter / 2) + 1; % T/2 point (1-indexed)

% Initialize data storage

model_data = cell(3, 1);

data_loaded = true;

% Read the model_data CSV files

for i = 1:3

try

% Read model_data CSV file

opts = detectImportOptions(model_data_files{i});

model_data{i} = readtable(model_data_files{i}, opts);

fprintf('Successfully loaded %s: %d rows\n', model_data_files{i}, height(model_data{i}));

catch ME

fprintf('Error loading %s: %s\n', model_data_files{i}, ME.message);

data_loaded = false;

break;

end

end

if ~data_loaded

error('Failed to load one or more CSV files. Please ensure the files are in the current directory.');

end

% Get unique observations and models

observations = unique(model_data{1}.observation);

models = unique(model_data{1}.model);

n_observations = length(observations);

n_models = length(models);

fprintf('Data dimensions: %d observations, %d models\n', n_observations, n_models);

fprintf('Half revolution starts at observation %d\n', half_revolution_start);

% Filter observations to only include those after T/2

observations_after_half = observations(observations >= half_revolution_start);

fprintf('Plotting %d observations after T/2\n', length(observations_after_half));

% Function to create ellipsoid points

function [x_ell, y_ell, z_ell] = create_ellipsoid(center, eigenvals, eigenvecs, confidence_level)

% Create unit sphere with fewer points for performance

[u, v] = meshgrid(linspace(0, 2*pi, 15), linspace(0, pi, 10));

x_sphere = sin(v) .* cos(u);

y_sphere = sin(v) .* sin(u);

z_sphere = cos(v);

% Scale by eigenvalues (standard deviations) and confidence level

x_scaled = x_sphere * sqrt(eigenvals(1)) * confidence_level;

y_scaled = y_sphere * sqrt(eigenvals(2)) * confidence_level;

z_scaled = z_sphere * sqrt(eigenvals(3)) * confidence_level;

% Rotate by eigenvectors

for i = 1:numel(x_scaled)

point = [x_scaled(i); y_scaled(i); z_scaled(i)];

rotated_point = eigenvecs * point;

x_scaled(i) = rotated_point(1);

y_scaled(i) = rotated_point(2);

z_scaled(i) = rotated_point(3);

end

% Translate to center

x_ell = x_scaled + center(1);

y_ell = y_scaled + center(2);

z_ell = z_scaled + center(3);

end

% Calculate trajectory and covariance for ALL models

model_trajectories = zeros(3, length(observations_after_half), n_models);

model_covariances = zeros(3, 3, length(observations_after_half), n_models);

for model_idx = 1:n_models

model_num = models(model_idx);

for obs_idx = 1:length(observations_after_half)

obs = observations_after_half(obs_idx);

% Get mean position for this model and observation

for state = 1:3

% Get data for this observation, state, and model

obs_data = model_data{state}(model_data{state}.observation == obs & ...

model_data{state}.model == model_num, :);

if ~isempty(obs_data)

model_trajectories(state, obs_idx, model_idx) = obs_data.mean(1);

model_covariances(state, state, obs_idx, model_idx) = obs_data.variance(1);

end

end

end

end

% Enhanced color schemes for better visualization

confidence_level = 2; % 2-sigma confidence level for better visibility

obs_step = max(1, floor(length(observations_after_half) / 6)); % Show every 6th observation

% Create multiple color schemes for different visualization layers

colormap_models = turbo(n_models); % Turbo colormap for models

colormap_time = parula(length(1:obs_step:length(observations_after_half))); % Time-based colors

%% MAIN COMPREHENSIVE VISUALIZATION

figure('Name', 'Comprehensive All-Models Visualization', 'Position', [50, 50, 1800, 1200]);

% Main 3D plot showing all models together

subplot(2, 3, [1, 2, 4, 5]);

hold on; grid on;

% Plot all model trajectories with different line styles and colors

line_styles = {'-', '--', '-.', ':'};

for model_idx = 1:n_models

traj = squeeze(model_trajectories(:, :, model_idx));

% Use different line styles for groups of models

line_style = line_styles{mod(model_idx-1, length(line_styles)) + 1};

% Plot trajectory with fade effect (darker to lighter)

for seg_idx = 1:size(traj, 2)-1

alpha_val = 0.3 + 0.7 * (seg_idx / size(traj, 2));

plot3(traj(1, seg_idx:seg_idx+1), traj(2, seg_idx:seg_idx+1), traj(3, seg_idx:seg_idx+1), ...

line_style, 'Color', [colormap_models(model_idx, :), alpha_val], ...

'LineWidth', 1.5);

end

% Plot ellipsoids for selected observations with varying transparency

for obs_idx = 1:obs_step:length(observations_after_half)

cov_matrix = squeeze(model_covariances(:, :, obs_idx, model_idx));

cov_matrix = cov_matrix + 1e-12 * eye(3);

[eigenvecs, eigenvals_diag] = eig(cov_matrix);

eigenvals = max(diag(eigenvals_diag), 1e-15);

center = model_trajectories(:, obs_idx, model_idx);

[x_ell, y_ell, z_ell] = create_ellipsoid(center, eigenvals, eigenvecs, confidence_level);

% Variable transparency based on model index and time

alpha_ell = 0.05 + 0.15 * (1 - model_idx/n_models) + 0.1 * (obs_idx/length(observations_after_half));

% Use different visualization techniques for different model groups

if model_idx <= n_models/3

% First third: Solid ellipsoids with low alpha

surf(x_ell, y_ell, z_ell, 'FaceColor', colormap_models(model_idx, :), ...

'FaceAlpha', alpha_ell, 'EdgeColor', 'none');

elseif model_idx <= 2*n_models/3

% Second third: Wireframe ellipsoids

surf(x_ell, y_ell, z_ell, 'FaceColor', 'none', ...

'EdgeColor', colormap_models(model_idx, :), 'EdgeAlpha', alpha_ell*2);

else

% Last third: Contour-style ellipsoids

contour3(x_ell, y_ell, z_ell, 5, 'Color', colormap_models(model_idx, :), ...

'LineWidth', 1);

end

% Plot center points with varying sizes

point_size = 4 + 8 * (model_idx/n_models);

plot3(center(1), center(2), center(3), 'o', 'Color', colormap_models(model_idx, :), ...

'MarkerSize', point_size, 'MarkerFaceColor', colormap_models(model_idx, :));

end

end

xlabel('X Position (km)'); ylabel('Y Position (km)'); zlabel('Z Position (km)');

title('All Models: Trajectories & Uncertainty Ellipsoids After T/2', 'FontSize', 14, 'FontWeight', 'bold');

view(45, 30); axis equal;

% Add colorbar to show model mapping

colorbar('Ticks', linspace(0, 1, min(10, n_models)), ...

'TickLabels', arrayfun(@(x) sprintf('M%d', x), 1:min(10, n_models), 'UniformOutput', false));

% Uncertainty evolution for all models

subplot(2, 3, 3);

hold on; grid on;

% Plot uncertainty evolution using different techniques

for model_idx = 1:n_models

uncertainty_x = sqrt(squeeze(model_covariances(1, 1, :, model_idx)));

uncertainty_y = sqrt(squeeze(model_covariances(2, 2, :, model_idx)));

uncertainty_z = sqrt(squeeze(model_covariances(3, 3, :, model_idx)));

% Total uncertainty (RMS)

total_uncertainty = sqrt(uncertainty_x.^2 + uncertainty_y.^2 + uncertainty_z.^2);

% Use different visualization for different model groups

if model_idx <= n_models/2

% First half: Filled areas

fill([observations_after_half; flipud(observations_after_half)], ...

[total_uncertainty; zeros(size(total_uncertainty))], ...

colormap_models(model_idx, :), 'FaceAlpha', 0.3, 'EdgeColor', 'none');

else

% Second half: Lines with different styles

line_style = line_styles{mod(model_idx-1, length(line_styles)) + 1};

plot(observations_after_half, total_uncertainty, line_style, ...

'Color', colormap_models(model_idx, :), 'LineWidth', 1.5);

end

end

xlabel('Observation Number');

ylabel('Total Uncertainty (km)');

title('All Models: Total Uncertainty Evolution', 'FontSize', 12, 'FontWeight', 'bold');

% XY Plane projection with all models

subplot(2, 3, 6);

hold on; grid on;

for model_idx = 1:n_models

traj = squeeze(model_trajectories(:, :, model_idx));

% Plot trajectory in XY plane

plot(traj(1, :), traj(2, :), 'Color', colormap_models(model_idx, :), 'LineWidth', 1.5);

% Plot ellipsoid projections

for obs_idx = 1:obs_step:length(observations_after_half)

cov_matrix = squeeze(model_covariances(:, :, obs_idx, model_idx));

cov_matrix_xy = cov_matrix(1:2, 1:2) + 1e-12 * eye(2);

[eigenvecs_xy, eigenvals_diag_xy] = eig(cov_matrix_xy);

eigenvals_xy = max(diag(eigenvals_diag_xy), 1e-15);

center_xy = model_trajectories(1:2, obs_idx, model_idx);

% Create 2D ellipse

theta = linspace(0, 2*pi, 30);

ellipse_unit = [cos(theta); sin(theta)];

ellipse_scaled = eigenvecs_xy * diag(sqrt(eigenvals_xy)) * ellipse_unit * confidence_level;

ellipse_xy = ellipse_scaled + center_xy;

% Different visualization for different models

if mod(model_idx, 3) == 1

plot(ellipse_xy(1, :), ellipse_xy(2, :), '-', 'Color', colormap_models(model_idx, :), 'LineWidth', 1);

elseif mod(model_idx, 3) == 2

plot(ellipse_xy(1, :), ellipse_xy(2, :), '--', 'Color', colormap_models(model_idx, :), 'LineWidth', 1);

else

fill(ellipse_xy(1, :), ellipse_xy(2, :), colormap_models(model_idx, :), 'FaceAlpha', 0.2, 'EdgeColor', 'none');

end

end

end

xlabel('X Position (km)'); ylabel('Y Position (km)');

title('All Models: XY Plane Projection', 'FontSize', 12, 'FontWeight', 'bold');

axis equal;

%% DETAILED STATISTICAL COMPARISON

figure('Name', 'All Models Statistical Analysis', 'Position', [100, 100, 1600, 1000]);

% Statistical comparison of all models

states = {'X', 'Y', 'Z'};

for state = 1:3

subplot(2, 3, state);

hold on; grid on;

% Create statistical plots for all models

uncertainties_all = zeros(length(observations_after_half), n_models);

for model_idx = 1:n_models

uncertainties_all(:, model_idx) = sqrt(squeeze(model_covariances(state, state, :, model_idx)));

end

% Plot individual models with transparency

for model_idx = 1:n_models

plot(observations_after_half, uncertainties_all(:, model_idx), ...

'Color', [colormap_models(model_idx, :), 0.6], 'LineWidth', 1);

end

% Plot statistical measures

mean_uncertainty = mean(uncertainties_all, 2);

std_uncertainty = std(uncertainties_all, 0, 2);

% Mean with confidence bands

fill([observations_after_half; flipud(observations_after_half)], ...

[mean_uncertainty + std_uncertainty; flipud(mean_uncertainty - std_uncertainty)], ...

[0.5, 0.5, 0.5], 'FaceAlpha', 0.3, 'EdgeColor', 'none');

plot(observations_after_half, mean_uncertainty, 'k-', 'LineWidth', 3);

plot(observations_after_half, mean_uncertainty + std_uncertainty, 'k--', 'LineWidth', 1);

plot(observations_after_half, mean_uncertainty - std_uncertainty, 'k--', 'LineWidth', 1);

xlabel('Observation Number');

ylabel('Position Uncertainty (km)');

title(sprintf('%s: All Models + Statistics', states{state}), 'FontWeight', 'bold');

end

% Model ranking by final uncertainty

subplot(2, 3, 4);

final_uncertainties = zeros(n_models, 3);

for model_idx = 1:n_models

for state = 1:3

final_uncertainties(model_idx, state) = sqrt(model_covariances(state, state, end, model_idx));

end

end

total_final_uncertainty = sqrt(sum(final_uncertainties.^2, 2));

[sorted_uncertainty, sort_idx] = sort(total_final_uncertainty);

bar(1:n_models, sorted_uncertainty, 'FaceColor', 'flat', 'CData', colormap_models(sort_idx, :));

xlabel('Model (sorted by performance)');

ylabel('Final Total Uncertainty (km)');

title('Model Ranking by Final Uncertainty', 'FontWeight', 'bold');

% Correlation matrix between models

subplot(2, 3, 5);

correlation_matrix = zeros(n_models, n_models);

for i = 1:n_models

for j = 1:n_models

uncertainty_i = sqrt(sum(squeeze(model_covariances(:, :, :, i)), 1));

uncertainty_j = sqrt(sum(squeeze(model_covariances(:, :, :, j)), 1));

corr_coeff = corrcoef(uncertainty_i, uncertainty_j);

correlation_matrix(i, j) = corr_coeff(1, 2);

end

end

imagesc(correlation_matrix);

colorbar;

xlabel('Model Index'); ylabel('Model Index');

title('Model Uncertainty Correlation Matrix', 'FontWeight', 'bold');

% Uncertainty spread analysis

subplot(2, 3, 6);

uncertainty_spread = std(uncertainties_all, 0, 2);

plot(observations_after_half, uncertainty_spread, 'b-', 'LineWidth', 2);

hold on; grid on;

% Add markers for key points

[max_spread, max_idx] = max(uncertainty_spread);

plot(observations_after_half(max_idx), max_spread, 'ro', 'MarkerSize', 10, 'MarkerFaceColor', 'r');

xlabel('Observation Number');

ylabel('Uncertainty Spread (km)');

title('Model Uncertainty Spread Over Time', 'FontWeight', 'bold');

%% SAVE ALL FIGURES

try

figure(1);

saveas(gcf, 'comprehensive_all_models_visualization.png', 'png');

saveas(gcf, 'comprehensive_all_models_visualization.fig', 'fig');

figure(2);

saveas(gcf, 'all_models_statistical_analysis.png', 'png');

saveas(gcf, 'all_models_statistical_analysis.fig', 'fig');

fprintf('All figures saved successfully\n');

catch ME

fprintf('Could not save figures: %s\n', ME.message);

end

% Enhanced summary statistics

fprintf('\n=== ENHANCED SUMMARY STATISTICS ===\n');

fprintf('Total models analyzed: %d\n', n_models);

fprintf('Observations after T/2: %d\n', length(observations_after_half));

fprintf('Ellipsoids per model: %d\n', length(1:obs_step:length(observations_after_half)));

% Calculate comprehensive statistics

fprintf('\n=== MODEL PERFORMANCE RANKING ===\n');

for i = 1:min(10, n_models)

model_idx = sort_idx(i);

fprintf('Rank %d: Model %d - Final uncertainty: %.3f km\n', ...

i, model_idx, sorted_uncertainty(i));

end

fprintf('\n=== UNCERTAINTY STATISTICS ===\n');

fprintf('Mean final uncertainty: %.3f ± %.3f km\n', mean(total_final_uncertainty), std(total_final_uncertainty));

fprintf('Best performing model: %d (%.3f km)\n', sort_idx(1), sorted_uncertainty(1));

fprintf('Worst performing model: %d (%.3f km)\n', sort_idx(end), sorted_uncertainty(end));

fprintf('\nComprehensive visualization complete!\n');

fprintf('All %d models shown with different visualization techniques\n', n_models);

fprintf('Multiple shading and styling techniques used for clarity\n');

% Modified MATLAB Script to Visualize Individual Model 3D Uncertainty Ellipsoids After T/2

% This script reads model_data CSV files and creates 3D ellipsoids for each model

% representing uncertainty only after T/2 (half revolution)

%------------------------for half ------------------

clear all; close all; clc;

% Define the CSV filenames for model data

model_data_files = {'model_data_x.csv', 'model_data_y.csv', 'model_data_z.csv'};

labels = {'X Position', 'Y Position', 'Z Position'};

% Orbital parameters (matching C++ code)

mu = 398600.0;

a = 8788.0;

T = 2.0 * pi * sqrt(a^3 / mu);

nObsPerIter = 24;

dtspan = T / nObsPerIter;

half_revolution_start = floor(nObsPerIter / 2) + 1; % T/2 point (1-indexed)

% Initialize data storage

model_data = cell(3, 1);

data_loaded = true;

% Read the model_data CSV files

for i = 1:3

try

% Read model_data CSV file

opts = detectImportOptions(model_data_files{i});

model_data{i} = readtable(model_data_files{i}, opts);

fprintf('Successfully loaded %s: %d rows\n', model_data_files{i}, height(model_data{i}));

catch ME

fprintf('Error loading %s: %s\n', model_data_files{i}, ME.message);

data_loaded = false;

break;

end

end

if ~data_loaded

error('Failed to load one or more CSV files. Please ensure the files are in the current directory.');

end

% Get unique observations and models

observations = unique(model_data{1}.observation);

models = unique(model_data{1}.model);

n_observations = length(observations);

n_models = length(models);

fprintf('Data dimensions: %d observations, %d models\n', n_observations, n_models);

fprintf('Half revolution starts at observation %d\n', half_revolution_start);

% Filter observations to only include those after T/2

observations_after_half = observations(observations >= half_revolution_start);

fprintf('Plotting %d observations after T/2\n', length(observations_after_half));

% Function to create ellipsoid points

function [x_ell, y_ell, z_ell] = create_ellipsoid(center, eigenvals, eigenvecs, confidence_level)

% Create unit sphere

[u, v] = meshgrid(linspace(0, 2*pi, 20), linspace(0, pi, 15));

x_sphere = sin(v) .* cos(u);

y_sphere = sin(v) .* sin(u);

z_sphere = cos(v);

% Scale by eigenvalues (standard deviations) and confidence level

x_scaled = x_sphere * sqrt(eigenvals(1)) * confidence_level;

y_scaled = y_sphere * sqrt(eigenvals(2)) * confidence_level;

z_scaled = z_sphere * sqrt(eigenvals(3)) * confidence_level;

% Rotate by eigenvectors

for i = 1:numel(x_scaled)

point = [x_scaled(i); y_scaled(i); z_scaled(i)];

rotated_point = eigenvecs * point;

x_scaled(i) = rotated_point(1);

y_scaled(i) = rotated_point(2);

z_scaled(i) = rotated_point(3);

end

% Translate to center

x_ell = x_scaled + center(1);

y_ell = y_scaled + center(2);

z_ell = z_scaled + center(3);

end

% Calculate trajectory and covariance for each model separately

model_trajectories = zeros(3, length(observations_after_half), n_models);

model_covariances = zeros(3, 3, length(observations_after_half), n_models);

for model_idx = 1:n_models

model_num = models(model_idx);

for obs_idx = 1:length(observations_after_half)

obs = observations_after_half(obs_idx);

% Get mean position for this model and observation

for state = 1:3

% Get data for this observation, state, and model

obs_data = model_data{state}(model_data{state}.observation == obs & ...

model_data{state}.model == model_num, :);

if ~isempty(obs_data)

model_trajectories(state, obs_idx, model_idx) = obs_data.mean(1);

model_covariances(state, state, obs_idx, model_idx) = obs_data.variance(1);

end

end

end

end

% Create figure for individual model ellipsoids

figure('Name', 'Individual Model 3D Uncertainty Ellipsoids After T/2', 'Position', [100, 100, 1400, 1000]);

% Configuration

confidence_level = 3; % 3-sigma confidence level

obs_step = max(1, floor(length(observations_after_half) / 8)); % Show every Nth observation

model_colors = hsv(n_models); % Different colors for each model

% Create subplots for different views

subplot(2, 2, 1);

hold on; grid on;

title('All Models - XY Plane View', 'FontSize', 12, 'FontWeight', 'bold');

subplot(2, 2, 2);

hold on; grid on;

title('All Models - XZ Plane View', 'FontSize', 12, 'FontWeight', 'bold');

subplot(2, 2, 3);

hold on; grid on;

title('All Models - YZ Plane View', 'FontSize', 12, 'FontWeight', 'bold');

subplot(2, 2, 4);

hold on; grid on;

title('All Models - 3D View', 'FontSize', 12, 'FontWeight', 'bold');

% Plot ellipsoids for each model

legend_entries = {};

plotted_observations = [];

for obs_idx = 1:obs_step:length(observations_after_half)

plotted_observations = [plotted_observations, observations_after_half(obs_idx)];

for model_idx = 1:min(n_models, 10) % Limit to first 10 models for clarity

% Get covariance matrix for this model and observation

cov_matrix = squeeze(model_covariances(:, :, obs_idx, model_idx));

% Add regularization to ensure positive definiteness

regularization = 1e-12;

cov_matrix = cov_matrix + regularization * eye(3);

% Eigenvalue decomposition

[eigenvecs, eigenvals_diag] = eig(cov_matrix);

eigenvals = diag(eigenvals_diag);

% Ensure positive eigenvalues

eigenvals = max(eigenvals, 1e-15);

% Center of ellipsoid

center = model_trajectories(:, obs_idx, model_idx);

% Create ellipsoid

[x_ell, y_ell, z_ell] = create_ellipsoid(center, eigenvals, eigenvecs, confidence_level);

% Plot in different subplots

% XY View

subplot(2, 2, 1);

plot(center(1), center(2), 'o', 'Color', model_colors(model_idx, :), ...

'MarkerSize', 6, 'MarkerFaceColor', model_colors(model_idx, :));

% XZ View

subplot(2, 2, 2);

plot(center(1), center(3), 'o', 'Color', model_colors(model_idx, :), ...

'MarkerSize', 6, 'MarkerFaceColor', model_colors(model_idx, :));

% YZ View

subplot(2, 2, 3);

plot(center(2), center(3), 'o', 'Color', model_colors(model_idx, :), ...

'MarkerSize', 6, 'MarkerFaceColor', model_colors(model_idx, :));

% 3D View

subplot(2, 2, 4);

surf(x_ell, y_ell, z_ell, 'FaceColor', model_colors(model_idx, :), ...

'FaceAlpha', 0.2, 'EdgeColor', 'none');

plot3(center(1), center(2), center(3), 'o', 'Color', model_colors(model_idx, :), ...

'MarkerSize', 6, 'MarkerFaceColor', model_colors(model_idx, :));

% Add to legend for first observation only

if obs_idx == 1

legend_entries{end+1} = sprintf('Model %d', model_idx);

end

end

end

% Style subplots

subplot(2, 2, 1);

xlabel('X Position (km)'); ylabel('Y Position (km)');

axis equal; grid on;

subplot(2, 2, 2);

xlabel('X Position (km)'); ylabel('Z Position (km)');

axis equal; grid on;

subplot(2, 2, 3);

xlabel('Y Position (km)'); ylabel('Z Position (km)');

axis equal; grid on;

subplot(2, 2, 4);

xlabel('X Position (km)'); ylabel('Y Position (km)'); zlabel('Z Position (km)');

view(45, 30); axis equal; grid on;

legend(legend_entries, 'Location', 'best', 'FontSize', 8);

% Create a separate figure for detailed model comparison

figure('Name', 'Detailed Model Comparison After T/2', 'Position', [200, 200, 1600, 1200]);

% Select a few representative models for detailed view

selected_models = 1:min(6, n_models);

subplot_rows = 2;

subplot_cols = 3;

for i = 1:length(selected_models)

model_idx = selected_models(i);

model_num = models(model_idx);

subplot(subplot_rows, subplot_cols, i);

hold on; grid on;

% Plot trajectory for this model

traj = squeeze(model_trajectories(:, :, model_idx));

plot3(traj(1, :), traj(2, :), traj(3, :), 'Color', model_colors(model_idx, :), ...

'LineWidth', 2, 'DisplayName', sprintf('Model %d Trajectory', model_idx));

% Plot ellipsoids for selected observations

for obs_idx = 1:obs_step:length(observations_after_half)

cov_matrix = squeeze(model_covariances(:, :, obs_idx, model_idx));

cov_matrix = cov_matrix + 1e-12 * eye(3);

[eigenvecs, eigenvals_diag] = eig(cov_matrix);

eigenvals = max(diag(eigenvals_diag), 1e-15);

center = model_trajectories(:, obs_idx, model_idx);

[x_ell, y_ell, z_ell] = create_ellipsoid(center, eigenvals, eigenvecs, confidence_level);

surf(x_ell, y_ell, z_ell, 'FaceColor', model_colors(model_idx, :), ...

'FaceAlpha', 0.3, 'EdgeColor', 'none');

plot3(center(1), center(2), center(3), 'o', 'Color', model_colors(model_idx, :), ...

'MarkerSize', 8, 'MarkerFaceColor', model_colors(model_idx, :));

end

xlabel('X (km)'); ylabel('Y (km)'); zlabel('Z (km)');

title(sprintf('Model %d (Index %d)', model_idx, model_num), 'FontWeight', 'bold');

view(45, 30); axis equal;

legend('show', 'Location', 'best');

end

% Create uncertainty evolution comparison

figure('Name', 'Model Uncertainty Evolution After T/2', 'Position', [300, 300, 1500, 500]);

states = {'X', 'Y', 'Z'};

for state = 1:3

subplot(1, 3, state);

hold on; grid on;

for model_idx = 1:min(n_models, 10) % Show first 10 models

uncertainty = sqrt(squeeze(model_covariances(state, state, :, model_idx)));

plot(observations_after_half, uncertainty, 'Color', model_colors(model_idx, :), ...

'LineWidth', 2, 'DisplayName', sprintf('Model %d', model_idx));

end

xlabel('Observation Number');

ylabel('Position Uncertainty (km)');

title(sprintf('%s Position Uncertainty (1-σ)', states{state}));

if state == 1

legend('show', 'Location', 'best', 'FontSize', 8);

end

end

sgtitle('Individual Model Uncertainty Evolution After T/2', 'FontSize', 14, 'FontWeight', 'bold');

% Save figures

try

figure(1);

saveas(gcf, 'individual_model_ellipsoids_after_T2.png', 'png');

saveas(gcf, 'individual_model_ellipsoids_after_T2.fig', 'fig');

figure(2);

saveas(gcf, 'detailed_model_comparison_after_T2.png', 'png');

figure(3);

saveas(gcf, 'model_uncertainty_evolution_after_T2.png', 'png');

fprintf('All figures saved successfully\n');

catch ME

fprintf('Could not save figures: %s\n', ME.message);

end

% Display summary statistics

fprintf('\n=== SUMMARY STATISTICS ===\n');

fprintf('Total models analyzed: %d\n', n_models);

fprintf('Observations after T/2: %d\n', length(observations_after_half));

fprintf('Ellipsoids plotted per model: %d\n', length(1:obs_step:length(observations_after_half)));

% Calculate average uncertainty for each model

fprintf('\nAverage uncertainty (1-σ) for each model after T/2:\n');

for model_idx = 1:min(n_models, 10)

avg_uncertainty = zeros(3, 1);

for state = 1:3

uncertainties = sqrt(squeeze(model_covariances(state, state, :, model_idx)));

avg_uncertainty(state) = mean(uncertainties);

end

fprintf('Model %d: X=%.3f km, Y=%.3f km, Z=%.3f km\n', ...

model_idx, avg_uncertainty(1), avg_uncertainty(2), avg_uncertainty(3));

end

fprintf('\nVisualization complete!\n');

fprintf('Generated individual model ellipsoids for %d models\n', min(n_models, 10));

fprintf('T/2 occurs at observation %d out of %d total observations\n', half_revolution_start, length(observations));