function main

% Constants

mu = 398600; % km^3/s^2

% Initial conditions

x0 = [-1871.0; 10639.0; 532.0; -6.2506; -1.0035; 0.1182];

delta_x = [-0.1871; 1.0639; 0.0532; -0.062506; -0.10035; 0.01182];

% Compute the orbital period

a = 12000; % semi-major axis (in km)

T = 2 * pi * sqrt(a^3 / mu); % Period in seconds

% Final state using RK4

xf = RK4(@TBP, x0, 0, T, mu);

% Output initial and final conditions

fprintf('Initial conditions (x0):\n');

print_vector(x0);

fprintf('Final conditions (xf) rounded to 6 digits:\n');

print_vector(round(xf, 6));

% Displaced initial conditions

x0_displaced = x0 + delta_x;

fprintf('Displaced Initial condition (x0 + delta_x):\n');

print_vector(x0_displaced);

% Displaced final conditions

xf_displaced = RK4(@TBP, x0_displaced, 0, T, mu);

fprintf('Displaced Final condition (xf evaluated at delta_x) rounded to 6 digits:\n');

print_vector(round(xf_displaced, 6));

end

function dx = TBP(~, x, mu)

% Two-body problem (TBP) function

pos = x(1:3); % position vector

vel = x(4:6); % velocity vector

r = norm(pos); % magnitude of position vector

% State derivatives

dx = zeros(6,1);

dx(1:3) = vel; % velocity components

dx(4) = -mu * pos(1) / (r^3); % acceleration in x direction

dx(5) = -mu * pos(2) / (r^3); % acceleration in y direction

dx(6) = -mu * pos(3) / (r^3); % acceleration in z direction

end

function xf = RK4(TBP, x0, t0, t1, mu)

% 4th order Runge-Kutta (RK4) method

hmax = 0.1; % maximum step size

steps = ceil((t1 - t0) / hmax); % number of steps

h = (t1 - t0) / steps; % step size

t = t0;

x = x0;

for i = 1:steps

% Compute the 4 Runge-Kutta stages

k1 = h * feval(TBP, t, x, mu);

k2 = h * feval(TBP, t + 0.5 * h, x + 0.5 * k1, mu);

k3 = h * feval(TBP, t + 0.5 * h, x + 0.5 * k2, mu);

k4 = h * feval(TBP, t + h, x + k3, mu);

% Update x for the next step using the RK4 formula

x = x + (1/6) * (k1 + 2 * k2 + 2 * k3 + k4);

% Increment time

t = t + h;

end

xf = x;

end

function print_vector(vec)

% Helper function to print vectors rounded to 6 digits

for i = 1:length(vec)

fprintf('%12.6f\n', round(vec(i), 6)); % Round to 6 digits

end

end