% Define the Keplerian elements

a = 12000;         % Semi-major axis in kilometers

e = .1;          % Eccentricity

i = 3;           % Inclination in degrees

Omega = 30;       % Longitude of the ascending node in degrees

omega = 60;       % Argument of periapsis in degrees

nu_full = linspace(0, 360, 1000);  % Full range of true anomaly from 0 to 360 degrees

nu_point = 10;     % True anomaly point to be highlighted (0 degrees)

% Convert angles to radians

i_rad = deg2rad(i);

Omega_rad = deg2rad(Omega);

omega_rad = deg2rad(omega);

nu_full_rad = deg2rad(nu_full);

nu_point_rad = deg2rad(nu_point);

% Calculate the initial position and velocity at nu_point using keplerian2ijk

[r0, v0] = keplerian2ijk(a, e, i_rad, Omega_rad, omega_rad, nu_point_rad);

y0 = [r0; v0];

mue = 398600.44189;  % Gravitational parameter in km^3/s^2

T =  2*pi * sqrt(a^3 / mue);  % Orbital period

tspan = [0 T];

% Solve the equations of motion using ode45 (Runge-Kutta 4/5)

odefun = @dfdf;

[t, y] = ode45(odefun, tspan, y0);  % Solve the ODE

% Extract position data from the ODE solution

xv = y(:,1);

yv = y(:,2);

zv = y(:,3);

% Find the final r and v at time t_final

r_final = y(end, 1:3);   % Final position vector (x, y, z) in km

v_final = y(end, 4:6);   % Final velocity vector (vx, vy, vz) in km/s

% Convert final r and v to Keplerian elements using ijk2keplerian

[a1, e1, i1, Omega1, omega1, nu_point1] = ijk2keplerian(r_final, v_final)

% Plot the initial orbit

figure;

hold on;

% Plotting initial orbit (calculated from Keplerian elements)

initial_orbit = [];

for j = 1:length(nu_full_rad)

[r, ~] = keplerian2ijk(a, e, i_rad, Omega_rad, omega_rad, nu_full_rad(j));

initial_orbit = [initial_orbit; r'];  % Store the orbit coordinates

end

plot3(initial_orbit(:,1), initial_orbit(:,2), initial_orbit(:,3), 'r', 'LineWidth', 2);

scatter3(r0(1), r0(2), r0(3), 'ro', 'DisplayName', 'Initial Position');

% Plot the final orbit (numerically solved)

plot3(xv, yv, zv, 'b', 'LineWidth', 2);

scatter3(r_final(1), r_final(2), r_final(3), 'bo', 'DisplayName', 'Final Position');

% Plot formatting

xlabel('X [km]');

ylabel('Y [km]');

zlabel('Z [km]');

title('Initial and Final Orbits');

legend('Initial Orbit', 'Initial Position', 'Final Orbit', 'Final Position');

grid on;

axis equal;

hold off;

%% ODE Function to compute the derivatives

function [dydt] = dfdf(t, y)

rv = y(1:3);           % Position vector

R = norm(rv);          % Magnitude of the position vector

mue = 398600.44189;    % Gravitational parameter

rdot = y(4:6);         % Velocity vector

rddot = -mue * rv / R^3;  % Acceleration vector

dydt = [rdot; rddot];  % Return [velocity; acceleration]

end

%% Function to convert Keplerian elements to position and velocity in IJK frame

function [r_ijk, v_ijk] = keplerian2ijk(a, e, i, Omega, omega, nu)

mue = 398600.44189;  % Gravitational parameter in km^3/s^2

% Calculate the distance r and the specific angular momentum h

r = a * (1 - e^2) / (1 + e * cos(nu));  % Distance from focus to the point

h = sqrt(mue * a * (1 - e^2));          % Specific angular momentum

% Position in the perifocal frame (PQW)

r_pqw = [r * cos(nu); r * sin(nu); 0];

% Velocity in the perifocal frame (PQW)

v_pqw = [-sqrt(mue/a) * sin(nu); sqrt(mue/a) * (e + cos(nu)); 0];

% Rotation matrix from perifocal (PQW) to IJK frame

R_pqw_to_ijk = [cos(Omega) * cos(omega) - sin(Omega) * sin(omega) * cos(i), -cos(Omega) * sin(omega) - sin(Omega) * cos(omega) * cos(i), sin(Omega) * sin(i);

sin(Omega) * cos(omega) + cos(Omega) * sin(omega) * cos(i), -sin(Omega) * sin(omega) + cos(Omega) * cos(omega) * cos(i), -cos(Omega) * sin(i);

sin(omega) * sin(i), cos(omega) * sin(i), cos(i)];

% Transform position and velocity to the IJK frame

r_ijk = R_pqw_to_ijk * r_pqw;

v_ijk = R_pqw_to_ijk * v_pqw;

end